Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:	  29 AUGUST 2018

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:         3 OCTOBER 2018

PROJECT:   1513 8th Ave SE; St. Cloud MN  Residential Roof Replacement

TO: 	         Qualified General Contractors

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for a residential roof replacement project , located at 1513 8th Ave SE; St. Cloud MN 56304.  Bids will be due Wednesday 3 October, 2018 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday 4 May 2018 at 8:30 AM.

A mandatory pre-bid site visit will be held onsite: Thursday September 20th 2017 at 2:00 PM.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Building Code (IBC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane. Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- Contractor shall be responsible for all debris removal related to all work performed under this work scope.

- Contractor to carry Builder’s Risk insurance on project for duration of construction.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Work Scope:

- All products per 2016 MLB Spec Book

- Tear-off and replace existing roofing.  Include Ice & water installation, replacement of underlayment, drip edge/gutter apron, valley metal flashings, roof jacks, all exhaust covers, ridge vent, and any other accessories which may be present.  Prep and paint any mechanical ventilation covers which will remain.  Owner to select new shingle color from Timberline stock colors.

- Tear off Facia and soffit.  Replace with aluminum fascia and soffit. Disconnecting and reinstalling light fixtures will be considered incidental to this activity

- Include replacement of 40 4ft by 8ft sheets of roof sheathing in the base bid.

- Include replacement of 40 lineal feet of 2 x 4 soffit dimensional lumber in base bid.

- Utilize Bid Alternate #1 for a unit cost per sheet installed to be credited back to Owner for any unused sheathing.  Additional sheets if necessary will also be based on this per sheet value.

- Utilize Bid Alternate #2 for a unit cost per lineal feet to be credited back to Owner for any unused soffit 2 x 4 dimensional lumber.   Additional feet if necessary will also be based on this per foot value.

- Permitting through Mille Lacs Band and City of St. Cloud, MN

Bid Alternate #1:  Unit Cost per sheet of roof sheathing installed.

Bid Alternate #2 for a unit cost per lineal feet to be credited back to Owner for any unused soffit 2 x 4 dimensional lumber.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Manager 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact Mike Moilanen - CMD Director of Project Manager at mike.moilanen@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within 30 days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Commercial Davis Bacon wage requirements (Mille Lacs County).

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - Letter from Surety Company stating companies’ ability to be bonded for the project.

  - A copy of Current Insurance Certificate

  - A copy of Subcontractor/Material Supplier list

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Craig Hansen at (320) 532-4778.

All proposals MUST be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: St. Cloud, MN Residential Roof Replacement

P.O. Box 509

Onamia, MN 56359

**Please note that the bids must be submitted via mail to the P.O. Box.

FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. **

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits and City of St. Cloud permits.

Licensing:

- Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. . Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@mlcv.com with questions regarding licensing and for the license application.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2018 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: ____________________________________________________________________________

JOB/PROJECT:     1513 8th Ave SE; St. Cloud MN  Residential Roof Replacement

LUMP SUM PRICE:

(Labor and materials):

___________________________________________________________________________$____________________

(Written Value)                                                                                                                    (Dollar Amount)

BID ALTERNATE #1:

(Unit price for additional 4ft by 8ft sheets of roof sheathing above the 40 included in the base bid)

__________________________________________________________________________$____________________

(Written Value)                                                                                                                  (Dollar Amount)

BID ALTERNATE #2:

(Unit price per foot of 2 x 4 dimensional lumber installed at soffit.  Include tear-off above the 40ft included in the base bid)

__________________________________________________________________________$____________________

(Written Value)                                                                                                                  (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 120 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- LETTER FROM BONDING SURETY

- COPY OF MINNESOTA CONTRACTORS LICENSE (if required)

- SUB-CONTRACTOR/SUPPLIER LIST (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: _____________________________________________________________

SECTION 07 3113 – ASPHALT SHINGLES

General Building Construction

PART 1  GENERAL

1.01	 SECTION INCLUDES

A.	Asphalt shingle roofing.

B.	Flexible sheet membranes for eave protection, underlayment, and valley protection.

C.	Associated metal flashings and accessories.

1.02	 RELATED REQUIREMENTS

A.	Section 06 1600 - Sheathing:  Roof sheathing.

B.	Section 07 6200 - Sheet Metal Flashing and Trim:  Edge and cap flashings.

1.03	 REFERENCE STANDARDS

A.	ASTM D 1970 - Standard Specification for Self-Adhering Polymer Modified Bituminous Sheet Materials Used as Steep Roofing Underlayment for Ice Dam Protection; 2001.

B.	ASTM D 3462 - Standard Specification for Asphalt Shingles Made From Glass Felt and Surfaced With Mineral Granules; 2007.

C.	ASTM D 4586 - Standard Specification for Asphalt Roof Cement, Asbestos-Free; 2007.

D.	ASTM D 4869 - Standard Specification for Asphalt-Saturated Organic Felt Underlayment Used in Steep Slope Roofing; 2005.

E.	NRCA MS104 - The NRCA Steep Roofing Manual; National Roofing Contractors Association; 2001, Fifth Edition, with interim updates.

F.	UL (RMSD) - Roofing Materials and Systems Directory; Underwriters Laboratories Inc.; current edition.

1.04	 QUALITY ASSURANCE

A.	Perform Work in accordance with the recommendations of NRCA Steep Roofing Manual.

B.	Installer Qualifications:  A firm or individual that is approved, authorized, or licensed by asphalt shingle roofing system manufacturer to install roofing system indicated.

C.	Products Required to Comply with Fire Resistance Criteria:  UL listed and labeled.

1.06	 DELIVERY, STORAGE, AND HANDLING

A.	Store roofing materials in a dry, well-ventilated, weather-tight location according to asphalt shingle manufacturer's written instructions.  Store underlayment rolls on end on pallets or other raised surfaces.  Do not double-stack rolls.

1.	Handle, store, and place roofing materials in a manner to avoid significant or permanent damage to roof deck or structural supporting members.

B.	Protect unused underlayment from weather, sunlight, and moisture when left overnight or when roofing work is not in progress.

1.07	 FIELD CONDITIONS

A.	Weather Limitations:  Proceed with installation only when existing and forecasted weather conditions permit asphalt shingle roofing to be performed according to manufacturer's written instructions and warranty requirements.

1.	Install self-adhering sheet underlayment within the range of ambient and substrate temperatures recommended by manufacturer.

1.08	 WARRANTY

A.	Special Warranty:  Manufacturer's standard form in which manufacturer agrees to repair or replace asphalt shingles that fail in materials or workmanship within specified warranty period.  Materials failures include manufacturing defects and failure of asphalt shingles to self-seal after a reasonable time.

1.	Material Warranty Period: 30 years from date of Substantial Completion, prorated, with first 10 years non-prorated.

2.	Wind-Speed Warranty Period:  Asphalt shingles will resist blow-off or damage caused by wind speeds up to 90 mph for 10 years from date of Substantial Completion.

3.	Algae-Discoloration Warranty Period:  Asphalt shingles will not discolor 10 years from date of Substantial Completion.

B.	Special Project Warranty:  Roofing Installer's warranty, on warranty form at end of this Section, signed by roofing Installer, covering Work of this Section, in which roofing Installer agrees to repair or replace components of asphalt shingle roofing that fail in materials or workmanship within the following warranty period:

1.	Warranty Period:  2 years from date of Substantial Completion.

PART 2  PRODUCTS

2.01	 SHINGLES

A.	Manufacturers:

1.	GAF Materials Corporation; Product – Advanced Protection Shingles  (PreviouslyTimberline 30 year):  www.gaf.com .

2.     No substitutes will be accepted.

B.	Asphalt Shingles:  Asphalt-coated glass felt, mineral granule surfaced, complying with ASTM D 3462;

1.	Self-sealing type.

2.	Style:  Square.

3.	Color:  As selected by CMD Project Management staff from manufacturer's full range.

2.02	 SOFFIT

A.	Aluminum Soffit:  Formed and coated aluminum soffit complying with AAMA 1402, including Paragraph 3.2.4.2.1, "Standard Commercial Coating."

1.	Basis of Design:  Rollex Corporation:  System 3.

2.	Other Acceptable Manufacturers:

a.	Alcoa Building Products, Inc.

b.	Gentek Building Products, Inc.

3.	Pattern:  12-inch exposure in V-grooved, triple 4-inch board style.

4.	Texture:  Smooth.

5.	Ventilation:  Provide perforated soffit where indicated.

a.	Free Air Space:  Minimum 9.72 square inches per lineal foot.

6.	Minimum Nominal Thickness:  0.019 inch.

7.	Finish:  Manufacturer's standard finish.

8.	Colors for Aluminum Soffit:  As selected by CMD Project Management staff from manufacturer's full range.

2.03	 SHEET MATERIALS

A.	Eave Protection Membrane:  Self-adhering polymer-modified asphalt sheet complying with ASTM D 1970; 40 mil total thickness; with strippable treated release paper and polyethylene sheet top surface.

1.	Manufacturers:

a.	CertainTeed Corporation; Product Winterguard.

b.	Grace, W. R. & Co.; Product Grace Ice and Water Shield.

c.	Johns Manville International, Inc.; Product Roof Defender.

d.	Owens Corning; Product WeatherLock M.

B.	Felt Underlayment:  Asphalt-saturated organic felt underlayment, complying with ASTM D 4869, minimum 15 lb/100 sq ft.

2.04	 ACCESSORIES

A.	Nails:  Standard round wire shingle type, of hot-dipped zinc coated steel, 12 gage, 0.105 inch shank diameter, 3/8 inch head diameter, of sufficient length to penetrate through roof sheathing or 3/4 inch into roof sheathing or decking.

B.	Ridge Vents:  Plastic, extruded with vent openings that do not permit direct water or weather entry; flanged to receive shingles; Shingle Vent II manufactured by Air Vent Inc.

2.05	 METAL FLASHINGS

A.	Metal Flashings:  Provide sheet metal eave edge, pre-formed metal valley, gable edge, ridge, ridge vents, open valley flashing, chimney flashing, dormer flashing, and other flashing indicated.

B.	Sheet Metal:  Prefinished galvanized steel, as specified in Section 07 6200.

PART 3  EXECUTION

3.01	 EXAMINATION

A.	Verify existing conditions prior to beginning work.

B.	Verify that deck is of sufficient thickness to accept fasteners.

C.	Verify that roof penetrations and plumbing stacks are in place and flashed to deck surface.

D.	Verify roof openings are correctly framed.

E.	Verify deck surfaces are dry, free of ridges, warps, or voids.

3.02	 PREPARATION

A.	Seal roof deck joints wider than 1/8 inch with deck tape.

B.	At areas where eave protection membrane is to be adhered to substrate, fill knot holes and surface cracks with latex filler.

C.	Broom clean deck surfaces before installing underlayment or eave protection.

3.03	 INSTALLATION - EAVE PROTECTION MEMBRANE

A.	Install eave protection membrane from eave edge to minimum 2 ft up-slope beyond interior face of exterior wall.

B.	Install eave protection membrane as indicated on drawings.

C.	Install eave protection membrane in accordance with manufacturer's instructions.

3.04	 INSTALLATION - UNDERLAYMENT

A.	Install underlayment perpendicular to slope of roof, with ends and edges weather lapped   minimum 4 inches. Stagger end laps of each consecutive layer. Nail in place. Weather lap minimum 4 inches over eave protection.

1.	Install felt underlayment on roof deck not covered by self-adhering sheet underlayment.  Lap sides of felt over self-adhering sheet underlayment not less than 3 inches in direction to shed water.  Lap ends of felt not less than 6 inches over self-adhering sheet underlayment.

B.	Items projecting through or mounted on roof:  Weather lap and seal watertight with plastic cement.

3.05	 INSTALLATION - VALLEY PROTECTION

A.	Install a 36-inch wide strip of ice/water protection lining centered in valley. Install pre-formed metal valley flashing and secure with roofing nails. Lap ends of strips at least 12 inches in direction to shed water, and seal with asphalt roofing cement.  Fasten to roof deck with roofing nails.

3.06 INSTALLATION - METAL FLASHING AND ACCESSORIES

A.	Install flashings in accordance with NRCA requirements.

B.	Weather lap joints minimum 2 inches and seal weather tight with plastic cement.

C.	Secure in place with nails at 8 inches on center.  Conceal fastenings.

D.	Items Projecting Through or Mounted on Roofing:  Flash and seal weather tight with plastic cement.

3.07	 INSTALLATION - SHINGLES

A.	Install shingles in accordance with manufacturer's instructions.

1.	Fasten individual shingles (hip/ridge) using 2 nails per shingle, or as required by code, whichever is greater.

2.	Fasten strip shingles using 4 nails per strip, or as required by code, whichever is greater.

B.	Place shingles in straight coursing pattern with 5 inch weather exposure to produce double thickness over full roof area.  Provide double course of shingles at eaves.

C.	Project first course of shingles 3/4 inch beyond fascia boards.

D.	Extend shingles 1/2 inch beyond face of gable edge fascia boards.

E.	Extend shingles on one slope across valley and fasten. Trim shingles 1” to 2” back from edge of pre-formed metal valley flashing.

F.	Cap ridges with individual shingles, maintaining 5 inch weather exposure.  Place to avoid exposed nails.

G.	Ridge Vents:  Install continuous ridge vents over asphalt shingles according to manufacturer's written instructions.  Fasten with roofing nails of sufficient length to penetrate sheathing.

H.    Mechanical through-roof venting to be installed on back side of roof (not visible from front street wherever possible)

I.	Ridge and Hip Cap Shingles:  Maintain same exposure of cap shingles as roofing shingle exposure.  Lap cap shingles at ridges to shed water away from direction of prevailing winds.  Fasten with roofing nails of sufficient length to penetrate sheathing.

1.	Fasten ridge cap asphalt shingles to cover ridge vent without obstructing airflow.

J.	Coordinate installation of roof mounted components or work projecting through roof with weather tight placement of counterflashings.

K..	Complete installation to provide weather tight service.

3.08	 PROTECTION

A.	Do not permit traffic over finished roof surface.

END OF SECTION