Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:    October 30, 2018

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:          November 28, 2018

PROJECT:      19241 364th Lane, McGregor

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for a residential renovation at 19241 364th Lane, McGregor, MN.   Bids will be due Wednesday November 28, 2018 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday November 29, 2018 at 8:30 AM.

A mandatory Pre-Bid site visit will be held on Tuesday, November 13, 2018 at 2:00 p.m.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description:

Exterior:

- Demo and replace all damaged siding and accessories. All replaced siding shall be color matched to existing. Use one-piece siding corners to cover damaged pieces; match to existing color.

- Contractor shall repair foundation flashing and finish covering exposed insulation on the rear of the home per Spec Book.

- Pressure wash and clean all siding and exterior surfaces.

- Demo and replace exterior light fixtures with spec product. Supply and install all new LED bulbs.

- Demo and replace overhead garage door weatherstripping and metal side jamb trim, match existing color.

- Demo and replace front entry door, frame and weatherstripping. Finish per Spec Book.

- Demo both sets of patio doors.

a.  Modify the lower opening to fit 3’ entry door per Spec Book. After modified, finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book.

b.  Modify the upper opening to install 72”x60” vinyl slider window. After modified, finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book.

- Supply and install spec storm doors to front and rear entries.

- Demo upper balcony and lower deck platform.

- Prep area and install a 12’x12’ concrete patio with #4 re-bar reinforcement @ 24” o.c. throughout for new 3’ rear entry door.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms.

- Supply and install missing or demo and replace damaged window screens.

- Replace missing or damaged gutter downspouts.

- Dispose of any existing splash pads. Contractor shall supply and install all new splashpads per the Spec Book.

- Demo the existing garage apron. Prep area with new clean sand level, compact base. Pour all new concrete to replace apron with #4 re-bar reinforcement @ 24” o.c. through and expansion joint between apron and garage floor. Sidewalk & garage apron height shall be poured to be within ¼” of the garage floor height.

- After existing apron is removed, install solid draintile for right side garage gutter downspout to run beneath new apron and daylight to east or west side of yard.

- Degrease and clean garage floor.

- Contractor shall supply and install 911 address sign above overhead garage door.

- Supply and install complete air conditioning system to include AC unit condenser, controller and lineset.

- Provide materials and labor to construct a 4’x4’ concrete pad 4” thick. Supply and install AC-Guard security cage model AC GU with AC-Guard Top Bar to be anchored with ½”x 3” Red Head anchor sleeves to the 4’x4’ pad.

- Demo and replace existing dryer vent.

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Install sheet rock for modified lower rear entry door opening. Texture, paint and finish, including installation of baseboard stained to match existing finish.

- Install sheet rock for modified opening where upper level rear entry door was removed. Texture, paint and finish around new window.

- Supply and install new entry door to garage, including frame.

- Supply and new lock sets on new front, rear and garage entry doors, as well as exterior garage service door; all to be keyed alike.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch and texture any damaged areas of drywall. Texturing to match existing as closely as possible. Prime and paint all interior drywall surfaces. Prime all drywall with Zinsser mold killing primer. Repaint Dover White. Apply dap to help conceal lower level ceiling crack before repainting.

- Demo and replace all interior light fixtures per Spec Book schedule, to include all new LED light bulbs.

- Demo and replace flooring throughout with spec sheet vinyl. To include supply and install spec underlayment. Vinyl selection to be 58024 Moonglow. Grind and prep subfloors as needed for a neat even finished appearance.

- Demo bi-fold closet doors on front entry closet. Supply and install double doors per Spec Book, stain and hardware to match existing finishes.

- Supply and install vinyl stair treads per Spec Book. Color selection: 29 Moon Rock.

- Supply and install new range hood vented to the exterior per Spec Book.

- Demo and replace all kitchen cabinet and island countertops. Selection shall be: 7267-58 Concrete Stone.

- Demo and replace kitchen sink base cabinet only. Sink and reuse existing sink and faucet.

- Thoroughly clean all kitchen cabinet surfaces. Supply and install missing shelving or hardware. All new hardware to match existing finishes. Adjust all cabinets.

- Demo and replace base-boards throughout with spec products. Base stain to match existing finishes in home. Spec vinyl base to be installed in bathrooms and mechanical room. Vinyl base color selection: 29 Moon Rock.

- Demo and replace all window extension jambs. To include spray foaming during installation.

- Demo and replace window casings with spec products, matching existing finishes.

- Thoroughly clean all window surfaces.

- Demo and replace both bath vanities with spec products reusing the existing tops, faucets and sinks. Thoroughly clean tops, sinks and faucets.

- Demo and replace; or supply and install all new bathroom accessories per Spec Book.

- Demo and replace showerheads with Moen product similar to Spec model.

- Demo and replace bath fans with spec products vented to the exterior.

- Thoroughly clean tubs and surrounds.

- Demo and replace toilet stools in both the upper and lower bathrooms.

- Re-caulk counter tops, toilet stool bases, tub/shower surrounds, as needed.

- Supply and install any missing or damaged door-stops throughout with spec product.

- Demo all existing closet shelving. Supply and install wire closet shelving per Spec Book throughout the home.

- Demo and replace all interior doors and hardware with spec products, doors stained to match existing finishes, hardware to match existing finish. Adjust all.

- Remove bi-fold doors, track cover trim and hardware from bedroom closets. No doors to be installed.

- Supply and install cap board on basement half walls; stain to match existing finishes.

- Service existing HRV unit.

- Demo and replace furnace per Spec Book. Clean and reinstall existing unused A-coil.

- Contractor shall supply and install new furnace thermostat model: Honeywell T6 Pro Series TH6210U2001.

- Contractor shall have all ductwork cleaned and provide certificate of completion.

- Demo and replace all floor vents with spec products.

- Thoroughly clean all wall registers, diffusers and return air covers.

- Demo and replace water heater with Spec model.

- Clean and paint mechanical/ laundry room floor per Spec Book.

- Insulate exposed plumbing pipes in mechanical room per Spec Book.

- Provide and install window blinds throughout with Spec product. Dispose of any existing.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact Ryan Jendro at 320-532-7857 or by email at ryan.jendro@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of  valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Craig Hansen at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: 19241 364th Lane, McGregor.

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

- Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@mlcv.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.  Site will also require city permitting.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2018 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  19241 364th Lane, McGregor

LUMP SUM PRICE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #1:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #1:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- LETTER FROM BONDING SURETY

- COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SUB-CONTRACTOR LISTS (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________