Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:      August 26, 2019

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:           September 12, 2019

PROJECT:      17687 Ookwemin Loop Fire Rehab

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for a residential fire damage renovation at 17687 Ookwemin Loop, Onamia, MN.   Bids will be due Wednesday September 11, 2019 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday September 12, 2019 at 8:30 AM.

A mandatory Pre-Bid site visit will be held on Thursday, August 29th, 2019, at 9:30 a.m.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description:

Exterior:

- Demo and replace fire damaged siding, house wrap, sheathing and framing members at the rear of the unit.

- Demo and replace all damaged siding and accessories. All replaced siding shall be color matched to existing.

- Pressure wash and or scrub exterior of building removing all dirt and soot.

- Demo and replace all damaged foundation flashing on the unit.

- Demo and replace exterior light fixtures per Spec Book.

- Supply and install all new LED light bulbs.

- Demo patio door. Modify the opening to fit 3’ entry door per Spec Book. Supply and install spec entry door with storm door. After modified, finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book.

- Contractor shall thoroughly clean and repaint existing front entry door slab.

- Supply and install new lock sets for front and rear entry doors per Spec Book; to be keyed alike.

- Supply and install spec storm doors to front and back entries with chain kits.

- Demo and replace all windows and screens.

- Demo and replace the soffit and fascia on the rear of the unit.

- Demo and replace damaged 6”x6” on front stoop. Demo and replace vinyl wrap.

- Contractor shall reset condenser properly on existing base.

- Contractor shall supply and install any missing outlet box covers.

Interior:

- Contractor shall demo all interior finishes, flooring, sheetrock, vapor barrier and insulation (wall and attic). Once everything is demolished, only the framing, electrical, ductwork and plumbing pipes shall remain inside.

- Contractor shall inspect electrical panel and breakers to ensure that all are functional. Contractor shall thoroughly clean panel cover.

- Contractor shall dispose of miscellaneous debris within the unit.

- Contractor shall demo and replace HRV, furnace with A-coil, water heater and laundry tub per Spec Book.

- Contractor shall supply and install new thermostat and HRV controller. Furnace thermostat model to be installed will be Honeywell T6 Pro Series TH6210U2001.

- Supply and install new spec AC controller and lineset for existing condenser.

- Contractor shall include a $2,500 allowance for roof rafter tail repairs, if required by the Building Inspector.

- Contractor shall demo and replace any damaged electrical components. Contractor shall include a $5,000 allowance for hidden wiring repairs.

- Contractor shall supply and install all spec combo smoke c/o detectors.

- Contractor shall have all ductwork cleaned.

- Contractor shall paint all framing with Kilz.

- Contractor shall supply and install all new insulation and vapor barrier. Attic insulation shall be insulated to R-49 and wall insulation shall be insulated to R-21.

- Contractor shall supply and install all new sheetrock, taping, texturing and painting per Spec Book.

- Contractor shall supply and install all new vinyl flooring throughout except the mechanical room. Vinyl selection is 58020 Oasis Beige.

- Clean and paint the mechanical room floor per the Spec Book.

- Contractor shall supply and install vinyl base in the bathroom and mechanical room. Vinyl selection is 09 Clay.

- Contractor shall supply and install all new interior doors (including hardware and door stops), casing, trim, and millwork throughout the unit. Stain selection is Early American 230.

- Contractor shall supply and install all new kitchen cabinets (to include all new hardware), countertops and bath vanities (to include all new hardware). Stain selection is Early American 230. Countertop finish is 3526-58 Traverine.

- Contractor shall supply and install all new wire shelving throughout.

- Contractor shall supply and install new range hood, oven and refrigerator per Spec Book.

- Contractor shall supply and install new kitchen and bath sinks and faucets; and new laundry tub faucet.

- Contractor shall supply and install a new toilet stool per Spec Book.

- Contractor shall supply and install a new tub/ shower per Spec Book.

- Contractor shall supply and install all new bath accessories.

- Contractor shall supply and install all new receptacles, wall plates and switch covers.

- Contractor shall supply and install all new grilles, diffusers and return air covers with spec products.

- Contractor shall supply and install all new floor registers.

- Contractor shall supply and install all new interior light fixtures per Spec Book schedule.

- Contractor shall supply and install all new LED bulbs throughout.

- Contractor shall supply and install all new blinds.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact either Carla Dunkley at 320-630-2495 or by email at carla.dunkley@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of  valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact TERO Office at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: 17687 Ookwemin Loop Fire Rehab

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@grcasinos.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2019 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  17687 Ookwemin Loop Fire Rehab

LUMP SUM PRICE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #1:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- LETTER FROM BONDING SURETY

- COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SUB-CONTRACTOR LISTS (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________