HIGHWAY 169 PEDESTRIAN CROSSING

MILLE LACS INDIAN RESERVATION

SUPPLEMENTAL SPECIFICATIONS TO THE

STATE OF MINNESOTA DEPARTMENT OF TRANSPORTATION

2018 STANDARD SPECIFICATIONS FOR CONSTRUCTION

These Special Provisions amend or supplement Division II and III of the Minnesota Department of Transportation (MN/DOT) Standard Specifications for Construction and the Materials Lab Supplemental Specifications for Construction, 2018 editions.  All work under this Contract shall be performed in accordance with Division II and III of the Minnesota Department of Transportation’s 2018 Editions, the latest supplemental specifications to the 2018 editions, the current Standard Detail Plates issued by MN/DOT, and these Special Provisions.

DIVISION I - GENERAL REQUIREMENTS & COVENANTS

Division I, General Requirements is in accordance with the 1996 Standard Specifications for Construction of Roads and Bridges on Federal Highway Projects (FP96), Division 100, General Requirements, and applies in its entirety, unless otherwise noted below.

# DIVISION II & III– CONSTRUCTION DETAILS & MATERIALS

2021

Mobilization

2021.1 Description

This work consists of moving personnel, equipment, materials, supplies, and incidentals to the project and performing all work necessary, before beginning and after completing work, at the project site along with the obtaining of all necessary permits, insurance, bonds and all of other work and facilities for work on the Project.

2105

Excavation and Embankment

2105.3	Construction Requirements

The Contractor shall be responsible for damage caused by his neglect to any underground utilities such as, but not limited to, gas mains, telephone and electric cables, water mains and sanitary sewer facilities.  The contractor shall notify the utility companies before work begins so as to be able to become familiar with the location of such facilities.

All topsoil encountered in the excavation for item 2501.503 (R.C. Pipe Culvert) shall be salvaged for use on the side slopes.

Compaction of embankment material on the side slopes shall be in accordance with Specification 2105, Subsection 2105.3F2, Quality Compaction Method.

The final shaping and compaction of the subgrade shall be done just prior to the construction of the gravel base.

2105.5	Basis of Payment

The subgrade preparation shall be considered incidental to the contract and no additional compensation allowed.

Topsoil Borrow:  Topsoil material which was salvaged as part of item 2501.503 (R.C. Pipe Culvert) shall be utilized prior to any topsoil borrow.  Use of topsoil borrow shall be at the Engineer’s discretion, and shall be placed a minimum of 4” in thickness or as directed by the Engineer.  This item will only be used in the event there is insufficient existing topsoil to salvage as determined by the engineer.  The salvaging and replacement of the existing topsoil is incidental and no additional payment will be made therefore.

After the replacement of the salvaged existing topsoil and prior to any topsoil borrow being purchased under this contract, the Engineer and the contractor will inspect the project and review the borrow quantity necessary to complete the project.  The Contractor shall not exceed this quantity without written authorization from the Engineer, or his/her representative.

Payment shall include all costs associated with the topsoil borrow acquisition, placement, hauling, all materials, labor, equipment, and other costs associated with this pay item.

“Topsoil Borrow Acceptance Schedule” is omitted.  The Engineer shall approve the topsoil borrow for acceptance.

2211

Aggregate Base

2211.2 Materials

The aggregate shall conform to the requirements of Specification 3138 for Class 5 aggregate.  All material sampling, testing, and inspecting will be monitored by the Owner or its agent at any time prior to or after being incorporated permanently in the work.

2211.3	Construction Requirements

Compaction of the aggregate base course shall be in accordance with Specification 2211, Subsection 2211.3D2b, Quality Compaction Method – Comply with requirements of 2105.3.F.2.

Prior to placing the aggregate base, the subgrade shall be shaped and compacted in accordance with Specification 2112, Subgrade Preparation.  This specification shall be applicable in its entirety except that payment for performing this work shall be incidental to the aggregate base and no direct payment shall be made.

The Contractor shall make his own arrangements for the materials required for the project under this Contract and shall make payment, if required, directly to the owner of the pit.

2357

Bituminous Tack Coat

2357.2 Materials

A  Bituminous Material delete the last sentence of the first paragraph and replace with the following:

The bituminous material for Tack Coat shall be cationic emulsion grade CSS-1H.

(2360)

SPECIFICATION SUPERPAVE HOT MIX ASPHALT (TYPE SP)

Mn/DOT 2350 and Mn/DOT 2360 are hereby deleted from the Mn/DOT Standard Specifications and replaced with the Combined 2360/2350 (Gyratory/Marshall Design) Specification, except as modified below.

Mix Designation Numbers for the bituminous mixtures on this Project is as follows:

Type SP 12.5 Wearing Course

SPWEB240B

Type SP 12.5 Non-Wearing Course

SPNWB240B

The final longitudinal joint shall be on the centerline of the road or as directed by the Engineer.

Pavement smoothness requirements of 2360.7C will not apply on this Project.  The requirements of 2360.7B (Surface Requirements) will apply.

Ordinary compaction method shall be used on this project.

2501

Pipe Culverts

2501 Aprons

The Pipe Aprons shall be installed in accordance with the applicable provisions of 2501 except as modified herein.

2501.3 Construction Requirements:

Mn/DOT specification 2451 shall apply to all bedding and backfill construction.  Compaction shall be obtained by the Quality Compaction Method.

2501.5 Basis of Payment:

The contract unit price shall include all of the following:  removal of existing c.s. pipe and aprons, excavation, pipe materials and appurtenances, approved pipe ties, circumferential wrap of geotextile material, embankment backfill as required, and all other work required to install the pipe complete in place.

2501 Pipe Culverts

The Pipe Culverts shall be installed in accordance with the applicable provisions of 2501 except as modified herein.

2501.3 Construction Requirements:

Mn/DOT specification 2451 shall apply to all bedding and backfill construction.  Compaction shall be obtained by the Quality Compaction Method.

2501.5 Basis of Payment:

The contract unit price shall include all of the following:  removal of existing c.s. pipe and aprons, excavation, pipe materials and appurtenances, approved pipe ties, circumferential wrap of geotextile material, embankment backfill as required, and all other work required to install the pipe complete in place.

2563

TRAFFIC CONTROL

All traffic control devices shall conform and be installed in accordance to the “Minnesota Manual on Uniform Traffic Control Devices” (MMUTCD) and Part VI, “Field Manual for Temporary Traffic Control Zone Layouts”, the “Guide to Establishing Speed Limits in Highway Work Zones”, the Minnesota Flagging Handbook, the provisions of Mn/DOT 1404 and 1710, the Minnesota Standard Signs Manuals Parts I, II, and III, the Traffic Engineering Manual Chapter 8 Appendixes 8-8.02 and 8-8.03, the Traffic Control Layouts/Typical Traffic Control Layouts in the Plans, and these Special Provisions.

The Contractor shall furnish, install, maintain, and remove all traffic control devices required to provide safe movement of vehicular traffic through the Project during the life of the Contract from the start of Contract operations to the final completion thereof.  The Engineer will have the right to modify the requirements for traffic control as deemed necessary due to existing field conditions.

Traffic control devices include, but are not limited to, barricades, warning signs, trailers, flashers, cones, drums, pavement markings and flagmen as required and sufficient barricade weights to maintain barricade stability.

- The Contractor shall furnish names, addresses, and phone numbers of at least three (3) individuals responsible for the placement and maintenance of traffic control devices.  These individuals shall be “on call” 24 hours per day, seven days per week during the times any traffic control devices, furnished and installed by the Contractor, are in place.  Two of these individuals must be available on the project within two hours of notification. The required information shall be submitted to the Engineer at the Pre-construction Conference.

- If the Contractor modifies the layout or sequence from the Plan, the Contractor shall submit the proposed traffic control layout to the Engineer, for approval, at least fourteen (14) days prior to the start of construction. At least 24 hours prior to placement, all traffic control devices shall be available on the project for inspection by the Engineer.  The Contractor shall modify his/her proposed traffic control layout and/or devices as deemed necessary by the Engineer.

- The Contractor shall inspect, on a daily basis, all traffic control devices, which the Contractor has furnished and installed, and verify that the devices are placed in accordance with the Traffic Control Layouts, these Special Provisions, and/or the MMUTCD.  Any discrepancy between the placement and the required placement shall be immediately corrected.

The Contractor shall be required to respond immediately to any call from the Tribe, Engineer or his designated representative concerning any request for improving or correcting traffic control devices.

2574

soil preparation

2574.507 Common Topsoil Borrow:  Topsoil material which was salvaged as part of excavation shall be utilized prior to any topsoil borrow.  Use of topsoil borrow shall be at the Engineer’s discretion, and shall be placed a minimum of 4” in thickness or as directed by the Engineer.  This item will only be used in the event there is insufficient existing topsoil to salvage as determined by the engineer.  The salvaging and replacement of the existing topsoil is incidental and no additional payment will be made therefore.

After the replacement of the salvaged existing topsoil and prior to any topsoil borrow being purchased under this contract, the Engineer and the contractor will inspect the project and review the borrow quantity necessary to complete the project.  The Contractor shall not exceed this quantity without written authorization from the Engineer, or his/her representative.

Payment shall include all costs associated with the topsoil borrow acquisition, placement, hauling, all materials, labor, equipment, and other costs associated with this pay item.

“Workmanship Rework Schedule” is omitted.  The Engineer shall approve the topsoil borrow for acceptance.

2575

Turf Establishment

2575.2	Materials

Seed, mix as specified

The seeding shall conform to the applicable requirements of Standard Specification 3876.  The seed mixture to be furnished and used shall be mixture 25-151 (seed rate 120lbs/acre).

2575.3	Construction Requirements

Seeding:  The work shall be performed in accordance with Standard specification 2575 except that no seeding shall be placed in the spring before April 15 or in the fall after October 15 without specific authority from the Contracting Officer.  Each separate area of seeding shall be watered and maintained in a condition satisfactory to the Tribe for a period of six weeks after the seeding is placed, except that no maintenance will be required in the fall after "freeze-up" or November 15, whichever is earlier.

Erosion Control Blankets shall be maintained in accordance with 2575, for a period of not less than six weeks.  Maintenance shall be incidental to the Erosion Control Blanket item.