Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:      May 12, 2020

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:           May 27, 2020

PROJECT:      1711 Linden Lane, Brainerd

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for a residential renovation at 1711 Linden Lane, Brainerd, MN.   Bids will be due Wednesday May 27th 2020 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday May 28th, 2020 at 8:30 AM.

A mandatory Pre-Bid site visit will be held Thursday May 21st at 10:00am at the site address listed above.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description:

Exterior:

- Tear-off existing roofing materials, house and attached garage.

- Remove roof sheathing at end of house gable end overhang. Install new framing members to existing roof trusses/rafters to correct the sagging/uneven roof line.  Install lookout blocking as needed at overhang to create a straight roof substrate and install new roof sheathing to even out with the existing roof.

- While roof is exposed, and access is available, contractor to blow-in an average of 4” of attic insulation.  Ensure soffit ventilation chutes are installed and air ventilation pathway between the exterior soffit overhang and the attic space is open to aid in ventilation air flow.

- Include up to 20 sheets of osb roof sheathing replacement.  Utilize bid Alternate #1 as a unit cost per sheet of sheathing installed.  Unused sheathing to be credited back to the Owner.

- Install new metal roof system, including Hi-Temp synthetic underlayment membrane metal roofing panels, rake end flashing covers, eave edge flashing / drip edge, and ridge cap.  Include replacement of existing roof penetration boot flashings, exhaust fan covers as needed for a complete installation.

- New metal roofing system to be: Metal Sales Image II Concealed Fastened system 26ga, 16” with minor ribs.  Color: Black

- Supply and install (2) Air Vent 12” black galvanized steel roof turbine vent assemblies.

- Demo and replace existing soffit intake vent covers.

- Supply and install additional soffit intake vent covers spaced evenly between the existing soffit vents to increase attic intake ventilation.

- Demo existing gutter system, including downspouts.  Fill exposed fastener holes.

- Install new metal fascia covers over existing fascia boards.  Homeowner to select color.

- Remove tree from front side yard identified by City of Brainerd.  Remove second tree from rear yard.  Trim additional tree limbs that present a threat to the home.  Trees will be identified at the pre-bid site visit.

- Contractor shall be responsible for all waste disposal and clean up associated with this work scope.

Interior:

- Repair damaged drywall ceiling failed tape joints.  Scrape texture, cut out failed tape, install new joint tape, compound and texture to match existing finish.  Paint ceiling areas as needed to blend to existing surrounding finish.

- Upper bathroom: Demo and replace bathroom vinyl flooring.  Include new doorway transition and vinyl base trim.  Homeowner to select colors.  Subfloor repair in front of the existing tub will be required.  Remove and re-install existing bathtub if needed to ease repair work.  Demo ceramic tile tub surround and wall wainscot.  Cut out and replace drywall tub surround, and skim coat drywall. Replace any damaged insulation and vapor barrier.  As wall cavities are opened up, install wood blocking for future installation of grab bars.  Repaint bathroom walls and ceiling.  Install new fiberglass shower panel wall system and shower curtain rod. Provide $250 material allowance for tub surround system.  Contractor to provide homeowner with selection information.

Bid Alternate #1:  Roof Sheathing Replacement Allowance.  Include 20 sheets of sheathing in the base bid.  Provide unit cost per sheet of sheathing in this Bid Alternate to calculate credit for unused installed product.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact either Ryan Jendro at 320.630.5011 or by email at ryan.jendro@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of  valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Craig Hansen at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: 1711 Linden LN

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@grcasinos.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits and City of Brainerd.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2019 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  1711 Linden Lane, Brainerd

LUMP SUM PRICE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #1: Unit Cost per sheet of roof sheathing

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- LETTER FROM BONDING SURETY

- COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SUB-CONTRACTOR LISTS (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________