Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:    July 1, 2020

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:           August 5, 2020

PROJECT:    East Lake Community Center Remodel

TO: 	         QUALIFIED GENERAL CONTRACTORS

________________________________________________________________________________________

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for a commercial remodel project to be performed at the East Lake Community Center located at 20930 367th Lane, McGregor, MN. Bids will be due Wednesday, August 5, 2020 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday August 6, 2020 at 9:00 AM.

A mandatory pre-bid site visit will be held on: Tuesday July 14, 2020 at 11:00 AM. Project plans and product specifications will be available to bidders at the pre-bid site visit.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Building Code (IBC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Project Narrative:

This project will consist of remodel of our East Lake Community Center.  Work will include general interior finish replacement, flooring, painting, ceiling tile, casework, interior wood door replacement, door hardware replacement, and light fixture replacement.  The existing gymnasium will receive new upper story window replacement, and synthetic overlay with surrounding LVT boarder flooring, and selective gymnasium equipment replacement.  Work is included in all public spaces as well as occupied offices, and our HHS clinical space. The anticipated work schedule will be late fall 2020 through winter.

Work Scope:

- Gymnasium Space:

- Replace basketball goal on the South end.

- Demo and replace athletic wall padding.  Contractor to provide Owner with samples for color selection.

- Prepare and install synthetic floor overlay, including gameline installation.  Contractor to provide Owner with samples for color selection

- Demo existing VCT gymnasium floor boarder and replace with LVT.  Prep and install floor leveler to meet the new synthetic flooring finished height.  New LVT may have a patterned design.  Owner’s building committee to provide design details.

- Clean, prep and paint steel base brackets securing the glue-lam assemblies.

- Demo and replace upper wood windows on East and West gymnasium.  New window assemblies to be fixed commercial aluminum assemblies.  Contractor shall be responsible for field measuring openings for replacement.  Provide and install exterior sealants as needed for a water tight installation.  Provide and install finished oak interior extension jambs and trim.

- Provide and install new double aluminum entry door storefront between the gymnasium and main foyer space.  Design and Finish to match existing Kawneer aluminum storefronts.  Door to include panic hardware and access control from the Foyer side into the gymnasium space.  Coordinate access control with existing building system.

- Entry Vestibules 100, 103, 200

- Demo and replace walk off mat carpet flooring.

- Demo and replace electric cabinet unit heaters.

- Repair/Replace any damaged door hardware and weather strip.

- Patch, prep and paint drywall ceilings.

- Provide and install new aluminum thresholds and door sweeps.

- Entry Vestibule 100: Supply and install automatic door openers, include push button switches.  Coordinate operators and switches with Owner.

- Entry Vestibule 100:  Re-Glaze side window glass in frame to fit properly.

- Entry Vestibule 100: Repair exterior damaged panel.

- Foyer 101, 106, 104.

- Demo and Replace VCT Flooring with LVT.

- Patch and paint drywall ceilings in 101, and 106.

- Rooms 409 & 410: These rooms were combined during construction and do not match the floor plan.

- Demo and Replace VCT Flooring with LVT, including vinyl base.

- Demo and Replace small sink base cabinet and top adjacent to the bathroom, include new sink and faucet.

- Replace kitchen sink faucet in 409.

- Remove and Replace Ceiling Tiles.

- Patch and paint all interior walls.

- Administration Area: 400, Offices 401, 402, 403, 404

- Remove and Replace Ceiling Tiles.

- Demo and replace casework in Administration 400 (reception desk and cabinet section)  Replace any damaged or affected carpet tiles affected by this work.

- Construct new casework for “mini” break area.  Consult with design committee group for casework detail.

- Include a material allowance of $30,000.00 in the base bid to cover the cost of casework fabrication.  Casework removal and installation labor to be included in the base bid.  Contractor to provide Owner with costs billed toward allowance.

- Janitor 102 and Mezzanine Stair Entry

- Demo VCT Flooring, prep concrete substrate and install epoxy paint finish.  Provide transitions to adjacent LVT flooring.

- Bathrooms 107 & 108

- Remove ceiling tile from grid system for re-installation.  Demo and replace the ceiling grid materials with corrosion resistant USG grid components for wet location.  Re-install ceiling tiles.  Replace any tiles that are damaged.

- HHS Clinic Space: Rooms 201, 202, 203, 204, 205, 206, 207, 300, 301, 302, 303, 304, 305, 306, 307, 308, 309

- Replace ceiling tile throughout area

- Demo and replace vinyl wall coverings where covered with vinyl wall covering.

- Patch and paint exposed drywall wall areas.

- Demo and replace sheet vinyl flooring in exam, lab, and dental areas.

- General Work: ALL AREAS.

- Demo and replace all interior flush wood doors.  New doors to be VT Industries flush wood veneer doors, Plain Sliced Red Oak or equal.  Provide pre-finished color options to Owner for selection.

- Remove hardware, sand, prep and paint all HM door frames.

- Demo and replace all interior door hardware, including hinges, kick plates, closers, locksets, stops, etc.  Contractor to provide Owner with finish options.  Keying to match existing configuration.  Match door hardware manufacturer and lockset functions to ensure building keying system is unchanged.

- Exterior wood window repairs:  Contractor shall be responsible for inspection and replacement of any damaged, broken, or non-functioning wood window components.  Contractor shall include a labor and material allowance of $5,000.00.  Prior to commencement of any repairs, contractor shall review a list of identified repairs with Owners representative to determine repair work approval.

- Demo and replace interior light fixtures.  Contractor shall include labor to remove and replace interior light fixtures in the base bid.  Contractor shall include a material allowance of $200,000.00.  This allowance shall be used for LED light fixture purchase.  Contractor is responsible for proving actual material cost and will be allowed a 10% markup on fixtures.  Contractor shall review existing lighting system and provide Owner with LED fixture replacement recommendations for review/approval.  The cost of the fixtures shall be paid from the allowance line.

- Interior Signage Allowance:  Contractor shall provide a $15,000.00 material allowance to remove existing and provide and install new interior signage. Labor for interior signage replacement shall be included in the base bid.

- Contingency Allowance:  Contractor shall include a contingency allowance line of $75,000.00 to be utilized by the Owner for project scope changes, unforeseen conditions or otherwise unspecified work.  Contractor shall submit any change request to the Owner for review/approval before commencing with any additional work that will be paid from the contingency allowance line.

- Exterior:

- Demo existing fencing surrounding the AC Condenser units.

- Prep, form and pour concrete maintenance pad.  (remove and re-install AC condensers as needed.  Ensure proper AC function upon re-installation.)

- Install 48” chain link fence surround with double gate for service.

Bid Alternate #1:   Exterior Concrete Replacement

Demo and replace concrete sidewalk from parking to entry 100 and entry 200.

Bid Alternate #2:   Domestic Water Treatment

Provide and install Iron filtration equipment to the existing water supply.  Contractor to size and connect in series prior to service entrance to the existing water softener system.

Bid Alternate #3:   Exterior Building lighting

Provide labor to demo and replace exterior building lighting, including wall packs and parking lot head replacement.  Fixture costs will be covered by the light fixture allowance.

Specified Product Substitutions:

All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact Ryan Jendro, Project Manager, at 320.630.5011 or at ryan.jendro@millelacsband.com to obtain bid form and to be included on the bidder’s list in the event that any addendums are issued for this project.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Surety Bond Letter

  - A copy of Subcontractor/Material Supplier list

  - A copy of  valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Lori Trail at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: East Lake Community Center Remodel Project

P.O. Box 509

Onamia, MN 56359

**Please note that the bids must be submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. **

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320) 532-8274 or via email at EThornbloom@grcasinos.com  with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- In construction contracts that are federally funded or deemed commercial, bonding is required.  These types of contracts shall demand a performance bond not less than twenty (20%) percent of the total contract price, but not to exceed $500,000.00.  A performance bond requirement is to ensure that, if a contractor defaults, the Band may request that the surety pay the expense incurred to complete the construction contract.

- In addition, all construction contracts identified as federally funded or commercial shall be covered by a payment bond equal to one payment installment or cover subcontractors/suppliers as determined by the Contracting Officer or his agents.  The payment bond must contain language stating that if the contractor fails to make a payment to its subcontractors/suppliers, the surety will make the necessary payment.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2020 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: ____________________________________________________________________________

JOB/PROJECT:    East Lake Community Center Remodel Project

BASE BID – (Labor and materials):

___________________________________________________________________________$____________________

(Written Value)                                                                                                                    (Dollar Amount)

ALTERNATE #1 Exterior Concrete Replacement

__________________________________________________________________________$____________________

(Written Value)                                                                                                                  (Dollar Amount)

ALTERNATE #2 Domestic Water Treatment

___________________________________________________________________________$____________________

(Written Value)                                                                                                                  (Dollar Amount)

ALTERNATE #3 Exterior Lighting

___________________________________________________________________________$____________________

(Written Value)                                                                                                                  (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- LETTER FROM BONDING SURETY (REQUIRED)

- COPY OF MINNESOTA CONTRACTORS LICENSE (if required)

- SUB-CONTRACTOR/SUPPLIER LIST (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: _____________________________________________________________