Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:   September 16, 2020

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:          September 30, 2020

PROJECT:      43572 Henry Davis Drive Onamia Renovation

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for a residential renovation at 43572 Henry Davis Drive, Onamia.   Bids will be due Wednesday September 30, 2020 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday October 1, 2020 at 8:30 AM.

A mandatory pre-bid site visit will be held on Monday, September 21, 2020 at 1:30 pm.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description:

- Contractor shall carefully jack the floor up and reset to level the center floor beam.

- Carefully remove and re-install any antenna / satellite tv mounts.  Consult with homeowner.

- Tear-off existing and replace roofing.  Include all roof trusses, sheathing, shingles, ice & water installation, replacement of underlayment , drip edge, valley metal flashings, roof jacks, soffit, fascia, gutters, bath fan exhaust covers, ridge vent / attic exhaust vents, step flashings and any other accessories which may be present.

- Demo and replace and ceiling drywall. New drywall shall be 5/8” gypsum board. Contractor shall supply and install a clear poly vapor barrier. Contractor shall prime, paint and finish ceiling to match existing but with a smooth finish.

- Remove and dispose of all existing attic insulation throughout the house.

- Contractor shall disconnect and reconnect all electrical wiring, as needed for ceiling and roof replacement.

- Contractor shall replace all roof jacks and exhaust vents penetrating the ceiling and roof and properly seal.

- Contractor shall supply and install all new ceiling register covers.

- Contractor shall construct a properly insulated attic access in the mechanical room.

- Demo and replace all interior light fixtures with Spec products, to include all new LED light bulbs per Spec Book schedule.

- Contractor shall supply and install new bath fan per Spec Book.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance

- Supply and install all new pre-manufactured roof trusses with a 12” energy heel and 5/12 pitch.

- Supply and install Simpson H25 Hurricane clips at each end of every roof truss.

- Insulate attic with blown fiberglass to meet R49 value.

- New roof shall have a continuous ridge vent.

- 1/150 roof vent at soffits and 1/150 vent at ridge.

- Supply and install an air chute at each truss space.

- Supply and install new ½” OSB sheathing for the roof deck.

- Supply and install new underlayment per the Spec Book.

- Supply and install ice and water membrane 24” past the exterior wall.

- Contractor shall supply and install all new drip edge.

- Contractor shall supply and install all new Timberline shingles per Spec Book. Owner to select new shingle color from Timberline stock colors.

- Contractor shall soffit/ fascia using 2”x6” sub fascia. Supply and install new maintenance free aluminum fascia cover and maintenance free aluminum vented soffit.

- Contractor should include cost of installing a LP Smart Siding Frieze Board, if needed to adjust for gap above existing siding; or sub fascia may need to be 2”x 8”. If frieze board is installed, its color shall match the aluminum fascia cover.

- On the gable ends, contractor shall install strip of sheathing for gap in truss height. Contractor shall install house wrap over new sheathing. Contractor shall supply and install new siding to infill the gap as needed, using as much of the existing siding as possible. Match new siding as close as possible to the existing.

- Contractor shall supply and install new gutters with leaf guards and downspouts per Spec Book.

- Contractor shall supply and install concrete splashpads per the Spec Book.

- Contractor shall demo and replace subfloor in the bathroom. Reinstall the existing toilet and caulk around stool base and at bottom of the existing tub.

- Contractor shall supply and install vinyl flooring, in the bathroom, per Spec Book. Vinyl selection shall be Oasis Beige.

- Contractor shall demo and replace existing bath vanity, sink and faucet; match existing finishes.

- Contractor shall demo and replace molding drywall between the bathroom and kitchen sink.

- Contractor shall demo and replace lower kitchen cabinet section including the countertop, sink, and faucet; match existing finishes.

- Contractor shall demo all existing windows; supply and install Thermotech vinyl sliding windows per the Spec Book.

- Adjust siding to fit new windows. Install j-channel around all per Spec Book. Caulk around perimeter.

- Contractor shall supply and install screens for all windows.

- Supply and install trim around newly installed windows after spray foam is completed around perimeter. Match to existing finishes.

- Contractor shall include a $4,000 allowance for repairing unknown or hidden damage.

Bid Alternate #1: Contractor shall provide deduct amount to only replace all broken glass panes on the existing windows.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact Carla Dunkley – CMD Project Coordinator at carla.dunkley@millelacsband.com or at 320.532.7429 to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Lori Trail at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: 43572 Henry Davis Drive Renovation

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@grcasinos.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2020 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  43572 Henry Davis Drive Renovation

LUMP SUM PRICE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

BID ALTERNATE #1 (Deduct to replace broken window glass only instead of all windows):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- LETTER FROM BONDING SURETY

- COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SUB-CONTRACTOR LISTS (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________