Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:   January 6, 2021

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:          February 3, 2021

PROJECT:      36564 208th Place & 20579 280th Street  McGregor MN

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for two residential renovations located at 36564 208th Place and 20579 380th Street, both McGregor MN.   Bids will be due Wednesday, February 3, 2021 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday February 4, 2021 at 8:30 AM.

A mandatory Pre-Bid site visit will be held on Wednesday, January 13, 2021 at 10:00 am. Site visit will start at 36564 208th Place and continue onto the 20579 280th Street house.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description: Unless otherwise noted all work shall be completed according to the 2016 MLB Spec Book.

36564 208th Place

Exterior:

- Use siding from the right gable end of the garage to patch and replace damaged siding on the house and garage. Use one-piece siding corners to cover damaged pieces; match to existing color. Remove crawl space vents and repair affected siding.

Demo and replace all damaged; or supply and install any missing siding, channel, louvered vents and accessories. All new shall match existing.

- Supply Certainteed Triple 3” smooth finish siding as close to matching existing siding color and install on the right gable end of the garage.

- Contractor shall remove and dispose of cable wires secured around the exterior of the house. Contractor shall caulk holes in the siding after removal with color matching caulk.

- Demo and replace all damaged; or supply and install any missing aluminum soffit and fascia covering. Remove vents installed in vented soffit. Replace affected pieces of soffit. All new pieces to match existing.

- Create metal trim corner covers to be riveted to the existing foundation coil stock and installed to repair damaged corners.

- Tear-off and replace existing house roofing.  Include ice & water installation, replacement of underlayment, drip edge, valley metal flashings, roof jacks, bath fan exhaust covers, ridge vent / attic exhaust vents, step flashings and any other accessories which may be present. Timberline shingles selection: Birchwood. Remove and dispose of satellite dish.

- Pressure wash and clean all siding and exterior surfaces.

- Demo and replace exterior light fixtures per Spec Book to include new LED light bulbs for all.

- Demo existing windows and replace with Spec Thermotech vinyl windows that meet egress requirements.

- Adjust siding to fit new windows. Install j-channel and 2” white brick mould around all per Spec Book. Caulk around perimeter.

- Supply and install all new window screens.

- Demo and replace front entry and both garage service doors and frames per Spec Book. Provide and install new lock sets. All to be keyed alike.

- Supply and install storm door with chain kit for front entry per Spec Book.

- Demo and replace decking, step, hand rails and posts for front entry. Vinyl wrap posts.

- Demo and replace damaged; or supply and install missing gutter and downspout components, including leaf guards.

- Contractor shall clean out all gutters.

- Supply and install concrete splash pads per Spec Book.

- Demo the existing garage apron. Prep area with new clean sand level, compact base. Pour all new concrete to replace apron with #4 re-bar reinforcement @ 24” o.c. through and expansion joint between apron and garage floor. Apron height shall be poured to be within ¼” of the garage floor height.

- Demo and replace overhead garage doors’ side jamb flashing and weatherstripping.

- Supply and install blue 911 sign above one of the garage overhead doors.

- Pour 4’x4’ concrete pad for rear garage door.

- Demo and replace all shutters. Selection: Mid-America 004 Wedgewood Blue.

- Supply and install complete air conditioning system to include AC unit condenser, controller and lineset.

- Provide materials and labor to construct a 5’x5’ concrete pad 4” thick. Supply and install AC-Guard security cage model AC GU with AC-Guard Top Bar to be anchored with ½”x 3” Red Head anchor sleeves to the 5’x5’ pad.

- Clean out all intake and exhaust vents.

- Contractor shall supply and install any missing outlet box covers.

- Supply and install any missing or damaged components to ensure proper function of exterior hose bibs.

- Remove and disposal of miscellaneous yard debris.

- Demo existing mailbox post; supply and install new post per Spec Book; and remount existing mailbox.

- Demo and replace well pump. Contractor shall shock existing well and test for E coli, coliform bacteria, nitrates and arsenic.

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Patch, texture, prime and paint all damaged interior drywall surfaces. Demo and replace any sections of drywall with heavy molding. Texturing to match existing as closely as possible, if not already textured. Prime all drywall with Zinsser mold killing primer. Repaint all drywall surfaces Dover White.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Remove existing ceiling fan. Supply and install new surface mount light fixture per Spec Book and controller.

- Demo and replace all interior light fixtures with Spec model surface mount fixture except recessed lighting and fixture in dining area.

- Demo and replace trim kits for recessed lighting.

- Supply and install all new LED light bulbs throughout per Spec Book schedule.

- Demo and replace any damaged; or supply and install any missing, damaged or painted receptacle and switch covers.

- Access attic; remove and replace any wet or damaged insulation and/or poly.

- Demo and replace flooring throughout with spec sheet vinyl except mechanical room floor, to include supply and install spec underlayment with floor leveler. Congoleum selection: Oasis Beige.

- Supply and install vinyl base boards in baths and mechanical rooms. Vinyl base selection: 29 Moon Rock.

- Demo and replace all kitchen cabinets and countertops. Match cabinets to existing finishes. Countertop selection: 7267-58 Concrete Stone.

- Demo and replace kitchen sink, faucet and side sprayer per Spec Book.

- Demo and replace range hood per Spec Book. Include venting to exterior and conceal.

- Supply and install new Whirlpool oven model# WFC150MOJW and refrigerator model# WRT311FZDW.

- Supply and install extension jambs and trim around newly installed windows after spray foam is completed around perimeter. Match to existing finishes.

- Supply and install spec window blinds per Spec Book.

- Demo and replace any damaged; or supply and install any missing baseboards, baseshoe, trim, jambs or casing. Woodwork should be free of water damage, cracking or excessive paint. Match to existing finishes.

- For each bathroom:

- Demo and replace bath fan with Spec model, vented to the exterior.

- Thoroughly clean and recaulk tub in main bath and shower in small bathroom.

- Demo and replace toilet stool per Spec Book.

- Demo and replace bath vanity, countertop, sink and faucet. Match vanity and cultured marble top per Spec Book to new kitchen cabinets.

- Demo and replace shower trim set.

- Demo and replace bath accessories per Spec Book.

- Demo and replace all existing; and supply and install wire shelving per Spec Bk throughout.

- Demo and replace all interior doors (except kitchen and bedroom closets), frames and hardware with spec product, doors stained to match existing finishes, hardware to have satin chrome finish. Doors shall all have knobs not levers.

- Supply and install spec door stops throughout.

- Demo existing kitchen and bedroom closet doors and frames. Repair sheetrock. Install trim and baseboard to match existing finishes for each opening.

- Demo existing and install new furnace with A-coil per Spec Book.

- Supply and install new furnace thermostat model: Honeywell T6 Pro Series TH6210U2001.

- Contractor shall have all ductwork cleaned.

- Demo and replace all floor vents, wall registers, diffusers and return air covers with spec products.

- Clean and paint mechanical/ laundry room floor per Spec Book.

- Insulate exposed plumbing pipes in mechanical room per Spec Book.

- Supply and install dryer vent box in mechanical room; and repair drywall opening for proper fitment. Paint and finish drywall repair.

- Service existing HRV unit.

- Demo and replace water heater with Spec model.

- Supply and install water softener per Spec Book with salt.

- Contractor shall complete water conditioning test and set softener hardness accordingly.

- Demo and replace well pressure tank in crawl space.

- Demo and replace; or supply and install new sump pump with hose extension long enough to direct discharge away from onsite septic system components.

- Remove and dispose of misc. debris in the home, garage and within crawl space.

- Thoroughly clean all interior surfaces.

- Include a $10,000 contingency allowance to be used at Project Coordinator’s discretion for needed repairs discovered after the project has commenced.

- Contractor shall include 0.5% TERO tax fee in base bid. Submission of receipt for paid TERO fee is required with application for final payment.

- Construction shall be permitted through the MLBO.

20579 380th Street

Exterior:

- Use one-piece siding corners to cover damaged pieces; match to existing color. Remove crawl space vents and repair affected siding. Demo and replace all damaged; or supply and install any missing siding, channel, louvered vents and accessories. All new shall match existing.

- Remove and dispose of cable wires secured around the exterior of the house. Contractor shall caulk holes in the siding after removal with color matching caulk.

- Demo and replace all damaged; or supply and install any missing aluminum soffit and fascia covering. All new pieces to match existing.

- Create metal trim corner covers to be riveted to the existing foundation coil stock and installed to repair damaged corners. Demo and replace damaged foundation flashing.

- Tear-off and replace existing house roofing.  Include ice & water installation, replacement of underlayment, drip edge, valley metal flashings, roof jacks, bath fan exhaust covers, ridge vent / attic exhaust vents, step flashings and any other accessories which may be present. Timberline shingles selection: Barkwood. Remove and dispose of satellite dish.

- Pressure wash and clean all siding and exterior surfaces.

- Demo and replace exterior light fixtures per Spec Book to include new LED light bulbs for all.

- Demo existing windows and replace with Spec Thermotech vinyl windows that meet egress requirements.

- Adjust siding to fit new windows. Install j-channel and 2” white brick mould around all per Spec Book. Caulk around perimeter.

- Supply and install all new window screens.

- Demo and replace shutters. Match existing color.

- Demo and replace front entry and both garage service doors, frames and brick mould per Spec Book. Provide and install new lock sets. All to be keyed alike.

- Demo patio door. Modify the opening, supply and install Thermotech 60”x42” slider window. After modified, finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book.

- Demo front entry storm door. Supply and install storm doors with chain kits for both front and rear entry per Spec Book.

- Demo and dispose of ramp. Supply and install spindles and railing to infill opening for now removed ramp. Demo and replace decking, stair treads, hand rails and any damaged spindles for front entry. Vinyl wrap posts.

- Demo and replace rear decking, stair treads, hand rails and any damaged spindles.

- Demo and replace damaged; or supply and install missing gutter and downspout components, including leaf guards.

- Contractor shall clean out all gutters.

- Supply and install concrete splash pads per Spec Book.

- Supply and install overhead garage door jamb flashing. Demo and replace weatherstripping and brick mould.

- Supply and install blue 911 sign above garage overhead door.

- Provide materials and labor to construct a 5’x5’ concrete pad 4” thick. Supply and install AC-Guard security cage model AC GU with AC-Guard Top Bar to be anchored with ½”x 3” Red Head anchor sleeves to the 5’x5’ pad.

- Contractor shall move existing A/C condenser and install onto concrete pad. Contractor may need to drain, disconnect, reconnect and/ or recharge condenser.

- Clean out all intake and exhaust vents.

- Contractor shall supply and install any missing outlet box covers.

- Supply and install any missing or damaged components to ensure proper function of exterior hose bibs.

- Remove and disposal of miscellaneous yard debris.

- Demo and replace well pump. Contractor shall shock existing well and test for E coli, coliform bacteria, nitrates and arsenic.

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Install sheet rock for modified rear entry door opening. Texture, paint and finish, including installation of baseboard and shoe stained to match existing finish.

- Patch, texture, prime and paint all damaged interior drywall surfaces. Texturing to match existing as closely as possible, if not already textured. Prime all new drywall with Zinsser mold killing primer. Repaint all new and damaged drywall surfaces Dover White.

- Supply and install all new smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Demo and replace damaged; or supply and install missing spindles on railing for steps inside the garage.

- Supply and install all interior light fixtures per Spec Book.

- Supply and install all new LED light bulbs throughout per Spec Book schedule.

- Demo and replace any damaged; or supply and install any missing, damaged or painted receptacle and switch covers.

- Supply and install missing subfloor in the living room.

- Demo any existing flooring. Supply and install new vinyl flooring throughout except mechanical room floor, to include supply and install spec underlayment with floor leveler. Congoleum selection: Sunstone.

- Supply and install vinyl base boards in baths and mechanical rooms. Vinyl base selection: 80 Fawn.

- Demo and replace all kitchen cabinets and countertops. Match cabinets to existing finishes. Countertop selection: 7736-58 Tuscan Marble.

- Demo and replace kitchen sink, faucet and side sprayer per Spec Book.

- Demo and replace range hood per Spec Book. Include venting to exterior and conceal.

- Supply and install new Whirlpool oven model# WFC150MOJW and refrigerator model# WRT311FZDW.

- Supply and install extension jambs and trim around newly installed windows after spray foam is completed around perimeter. Match to existing finishes.

- Supply and install spec window blinds per Spec Book.

- Supply and install all baseboards, baseshoe, trim, jambs and casing. Match to existing stain finishes.

- For each bathroom:

- Demo and replace bath fan with Spec model, vented to the exterior.

- Supply and install H330/331 FF Tub/Shower in main bath. Demo shower stall in smaller bath and replace with H 852/ 853.

- Supply and install toilet stool per Spec Book.

- Supply and install bath vanity, countertop, sink and faucet. Vanity and cultured marble top per Spec Book to match new kitchen cabinets. (Vanity sizing should the same as the ones located in 36564 208th Place.)

- Supply and install shower trim set.

- Supply and install bath accessories per Spec Book.

- Demo and replace all existing; and supply and install wire shelving per Spec Bk throughout.

- Supply and install all pre-hung interior doors (except kitchen and bedroom closets), frames and hardware with spec product, doors stained to match existing finishes, hardware to have satin chrome finish. Doors shall receive knobs not levers.

- Supply and install spec door stops throughout.

- For kitchen corner and bedroom closet openings, install trim and baseboard to match existing finishes for each opening.

- Service existing furnace and HRV unit.

- Demo existing thermostat and replace with model: Honeywell T6 Pro Series TH6210U2001.

- Contractor shall have all ductwork cleaned.

- Supply and install all floor vents, wall registers, diffusers and return air covers spec products.

- Clean and paint mechanical/ laundry room floor per Spec Book.

- Insulate exposed plumbing pipes in mechanical room per Spec Book.

- Supply and install water heater per Spec Book.

- Supply and install water softener per Spec Book with salt.

- Contractor shall complete water conditioning test and set softener hardness accordingly.

- Demo and replace well pressure tank in crawl space.

- Demo and replace; or supply and install new sump pump with hose extension long enough to direct discharge away from onsite septic system components.

- Remove and dispose of misc. debris in the home, garage and within crawl space.

- Thoroughly clean all interior surfaces.

- Include a $10,000 contingency allowance to be used at Project Coordinator’s discretion for needed repairs discovered after the project has commenced.

- Contractor shall include 0.5% TERO tax fee in base bid. Submission of receipt for paid TERO fee is required with application for final payment.

- Construction shall be permitted through the MLBO.

Bid Alternate #1 (20579 380th St.): Supply difference in cost to complete the following instead of repairing the existing rear deck as described in exterior item #16.

Contractor shall demo deck and concrete landing. Contractor supply and install fill and prep area to pour 12’x12’ patio slab per Spec Book. Contractor shall install #4 re-bar reinforcement @ 24” o.c. Install expansion joint between foundation and patio slab. Contractor shall construct 4’x4’ wood landing with one (1) 4’x12” step down, to be properly anchored to the concrete patio. Rear entry door shall be centered with patio slab. Contractor shall finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Note: Depending on bid results, the Owner reserves the right to divide the rehabs into individual projects, if it is in the Band’s financial best interest to do so.

Contacts:

Interested bidders shall contact Carla Dunkley at 320-630-2495 or by email at carla.dunkley@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount.

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - Completed schedule of values (billing worksheet page 2) for each house.

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of  valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Lori Trail at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: 36564 208th Place & 20579 380th Street

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@grcasinos.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2021 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  36564 208th Place & 20579 380th St., McGregor MN

LUMP SUM PRICE (36564 208th Place):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

LUMP SUM PRICE (20579 380th Street):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

LUMP SUM PRICE (Both homes):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

BID ALTERNATE #1 (20579 380th St Deck Demo & Patio Slab):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- LETTER FROM BONDING SURETY

- COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SUB-CONTRACTOR LISTS (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________