Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:    February 10, 2021

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:        March 11, 2021

PROJECT:      44885 Biboon Ct Duplex & 42938 Timber Trails Rd, Onamia Rehabs

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for 2 residential renovations located at 44885 Biboon Ct and 42938 Timber Trails Rd, Onamia.   Bids will be due Wednesday, March 10, 2021 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday, March 11, 2021 at 8:30 AM.

A mandatory Pre-Bid site visit will be held on February 18, 2021 at 11:00 a.m. Site visit shall start at 44885 Biboon Ct then continue onto 42938 Timber Trails Rd.

If you are interested but unable to attend the January 11th site visits, contact Carla Dunkley at 320-630-2495 or via email Carla.Dunkley@millelacsband.com prior to February 18, 2021 to inquire about scheduling a make-up site visit.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description: Unless otherwise noted all work shall be completed according to the 2016 MLB Spec Book.

44885 Biboon Ct Duplex

Exterior (Applies to Units A & B unless noted otherwise):

- Demo and replace all damaged; or supply and install any missing siding, channel, louvered vents and accessories. All new siding shall match existing.

- Demo and replace all damaged; or supply and install any missing aluminum soffit and fascia covering. All new pieces shall match existing.

- Demo and replace damaged post wraps. Match existing.

- Demo and replace damaged foundation flashing.

- Supply and install black dirt to correct perimeter grade, especially along the front of the building. No insulation shall be exposed.

- Contractor shall regrade any ruts in the yard. Supply black dirt and seed after corrected.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms.

- Supply and install missing; or demo and replace damaged window screens.

- Pressure wash, scrub and clean all siding and exterior surfaces.

- Remove and dispose of cable wires secured around the exterior of the house. Caulk holes in the siding after removal with color matching caulk.

- Remove and dispose of the existing dishes installed on the roof. Caulk screw holes or repair any damage to the roof.

- Demo and replace damaged; or supply and install missing Spec Bk exterior light fixtures.

- Supply and install all new LED bulbs for all exterior light fixtures.

- Demo and replace damaged; or supply and install missing gutter and downspout components, including leaf guards.

- Contractor shall clean out all gutters.

- Demo and replace any damaged concrete splashpads per Spec Book. After grade corrections, reset splashpads so that they are all properly aligned.

- Demo and replace front entry door and frame per the Spec Book, include new aluminum clad wrap and brick mould.

- Demo and replace rear entry door and frame per the Spec Book. include new aluminum clad wrap and brick mould.

- Supply and install new lock sets for both front and rear entry doors. All entry door lock sets to be keyed alike.

- Demo existing storm doors. Supply and install spec storm doors to front and back entries with chain kits.

- Thoroughly clean dividing fence and repaint to match granite grey siding.

- Supply and install any missing outlet box covers.

- Contractor shall clean out intake and exhaust vents.

- Supply and install any missing or damaged components to ensure proper function of exterior hose bibs.

- Provide materials and labor to construct a 5’x5’ concrete pad 4” thick. Supply and install AC-Guard security cage model AC GU with AC-Guard Top Bar to be anchored with ½”x 3” Red Head anchor sleeves to the 5’x5’ pad.

- Contractor shall move existing A/C condenser and install onto concrete pad. Contractor may need to drain, disconnect, reconnect and/ or recharge condenser.

- Unit B Only – Supply and install cover for A/C disconnect box.

Interior (Applies to Units A & B unless noted otherwise):

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Remove and dispose of any remaining belongings or trash within the house.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all damaged interior drywall surfaces. Demo and replace any sections of drywall with heavy molding. Sand any existing drywall areas of texture or seams which do not blend-in well. All (new and existing) texturing to match as closely as possible. Prime all drywall with Zinsser mold killing primer. Repaint all drywall surfaces Dover White.

- Demo and replace any damaged; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace all grilles, diffusers and return air covers with spec products.

- Demo and replace any damaged; or supply and install any missing interior light fixtures with Spec products, to include all new light bulbs per Spec Book schedule.

- Install new LED bulbs throughout the interior.

- Demo and replace flooring throughout. New vinyl shall be Oasis Beige.

- Demo and replace lower kitchen cabinets and countertops. Note: Unit B’s existing cabinetry is missing the outer countertop. Supply and install new knobs to match existing. Demo and replace damaged; or supply and install missing upper cabinet components.  Stain finish and hardware to match existing. Thoroughly clean all cabinets.

- Demo and replace kitchen sink, faucet and side sprayer per Spec Book.

- Demo existing range hood and replace with Spec model.

- Unit A Only – Remove and dispose of spice rack mounted on wall.

- Supply and install new Whirlpool oven model# WFC150MOJW and refrigerator model# WRT311FZDW.

- Demo and replace any damaged; or supply and install missing baseboards, baseshoe, trim, jambs or casing. Woodwork should be free of water damage, cracking or excessive paint. Match to existing finishes.

- Demo and replace all lower window extension jambs. Include spray foam during installation. Match existing finishes.

- Demo and replace any damaged; or supply and install any missing window components.

- Thoroughly clean all window surfaces.

- Demo and replace any damaged interior pre-hung doors and frames. Match existing finishes.

- Adjust all interior doors.

- Demo and replace all door levers; or supply and install new door knobs. Match existing finishes. Note: double closet doors should have a knob on each.

- Demo and replace damaged; or supply and install missing door hardware, hinges, stops or strike plates. Match existing finishes.

- Demo and replace bath vanity and top. Supply and install new knobs. Match existing finishes and hardware.

- Demo and replace bath faucet.

- Demo and replace toilet stool.

- Thoroughly clean tub/ shower and recaulk.

- Demo and replace shower trim set.

- Demo and replace bath fan with Spec model.

- Demo and replace damaged; or supply and install missing bath accessories per Spec Book.

- Demo and replace damaged; or supply and install missing wire shelving per Spec Book.

- Service existing furnace and HRV. Ensure that filters are changed; or supply and install new filters.

- Demo and replace laundry tub faucet.

- Ensure proper function of water meter.

- Thoroughly clean mechanical room floor and repaint.

- Demo and replace any damaged or loose; or supply and install all missing vinyl base. Match existing.

- Provide and install window blinds throughout with Spec product. Dispose of any existing.

- Thoroughly clean all interior surfaces.

- Contractor shall have all ductwork cleaned.

- Include a $7,500.00 contingency allowance (per duplex side) to be used at Project Coordinator’s discretion for needed repairs discovered after the project has commenced.

- Contractor shall include 0.5% TERO tax fee in base bid. Submission of receipt for paid TERO fee is required with application for final payment.

- Construction shall be permitted through the MLBO.

42938 Timber Trails Rd

Exterior:

- Demo and replace all damaged; or supply and install any missing siding, channel, louvered vents and accessories. All new siding shall match existing.

- Demo and replace all damaged; or supply and install any missing aluminum soffit and fascia covering. All new pieces shall match existing.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms.

- Supply and install missing; or demo and replace damaged window screens.

- Pressure wash, scrub and clean all siding and exterior surfaces.

- Remove and dispose of cable wires secured around the exterior of the house. Caulk holes in the siding after removal with color matching caulk.

- Remove and dispose of the existing dishes installed on the roof. Caulk screw holes or repair any damage to the roof.

- Demo existing exterior light fixtures. Supply and install all new Spec Bk exterior light fixtures.

- Supply and install all new LED bulbs for all exterior light fixtures.

- Demo and replace damaged; or supply and install missing gutter and downspout components, including leaf guards.

- Contractor shall clean out all gutters.

- Demo and replace any damaged concrete splashpads per Spec Book.

- Demo and replace overhead garage door, metal jamb flashing and weatherstripping. Demo and replace friezeboard flashing. Match existing colors.

- Demo and replace front entry door and frame per the Spec Book, include new aluminum clad wrap and brick mould.

- Demo patio door. Modify the opening to fit 3’ entry door per Spec Book. Supply and install spec entry door and frame, including wider jamb aluminum clad wrap (match to front entry) and brick mould. After modified, finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book.   Note: remember to figure cost of thicker framing to match ICF wall.

- Supply and install new lock sets for both front and rear entry doors. All entry door lock sets to be keyed alike.

- Demo existing front storm door. Supply and install spec storm doors to front and back entries with chain kits.

- Supply and install any missing outlet box covers.

- Contractor shall clean out intake and exhaust vents.

- Supply and install any missing or damaged components to ensure proper function of exterior hose bibs.

- Provide and install AC condenser, lineset and controller.

- Demo and dispose of existing AC pad. Provide materials and labor to construct a 5’x5’ concrete pad 4” thick. Supply and install AC-Guard security cage model AC GU with AC-Guard Top Bar to be anchored with ½”x 3” Red Head anchor sleeves to the 5’x5’ pad.

- Driveway – Contractor shall supply and install reclaimed asphalt over the top of the existing driveway. Newly placed topping shall be of the same width as the existing asphalt. Depth of topping shall vary but shall be installed to create a level finished driveway that provides for a smooth ride. Coordinate driveway completion so that Project Coordinator can be onsite for inspection and provide additional direction.

- Remove and dispose of deck and supports for loft on detached garage. Board the patio door shut.

- Well - Contractor shall be responsible for disinfecting the existing well; and testing for coliform bacteria and nitrates. Demo and replace well pump.

- Septic – Contractor shall mow mound and cut small trees growing on it flush to the surface.

- Mailbox – Contractor shall provide and install mailbox per Spec Bk.

- Contractor shall dispose of miscellaneous yard debris.

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Remove and dispose of any remaining belongings or trash within the house.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Install sheet rock for modified rear entry door opening. Texture, paint and finish, including installation of baseboard and shoe stained to match existing finish.

- Patch, texture, prime and paint all damaged interior drywall surfaces. Demo and replace any sections of drywall with heavy molding. Sand any existing drywall areas of texture or seams which do not blend-in well. All (new and existing) texturing to match as closely as possible. Prime all drywall with Zinsser mold killing primer. Repaint all drywall surfaces Dover White.

- Demo and replace garage service door and frame. Demo and replace door lock set. To be keyed alike all entry doors.

- Remove and dispose of shelving in the garage.

- Demo and replace any damaged; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace damaged; or supply and install missing grilles, diffusers and return air covers with spec products.

- Demo and replace any damaged; or supply and install any missing interior light fixtures with Spec products, to include all new light bulbs per Spec Book schedule. All interior light fixtures shall be in accordance with the Spec Bk.

- Install new LED bulbs throughout the interior.

- Demo ceiling fan in living room. Supply and install Samantha 3 light semi-flush mounted light fixture model: cf57517sn and controller per Spec Bk.

- Demo and replace flooring throughout. New vinyl shall be 58021 Sunstone.

- Remove and dispose of existing dishwasher.

- Demo and replace all lower kitchen cabinets and countertop. New cabinets shall include 24” cabinet to replace existing dishwasher. Supply and install new knobs to match existing. Demo and replace damaged; or supply and install missing upper cabinet components, including front cover for venting conceal.  Stain finish and hardware to match existing.

- Demo and replace kitchen sink, faucet and side sprayer per Spec Book.

- Demo existing range hood and replace with Spec model.

- Supply and install new Whirlpool oven model# WFC150MOJW and refrigerator model# WRT311FZDW.

- Demo and replace any damaged; or supply and install missing baseboards, baseshoe, trim, jambs or casing. Woodwork should be free of water damage, cracking or excessive paint. Match to existing finishes.

- Sand and refinish all bottom window extension jambs. Stain to match existing finish.

- Demo and replace any damaged; or supply and install any missing window components.

- Thoroughly clean all window surfaces.

- Demo and replace any damaged interior pre-hung doors and frames. Match existing finishes.

- Adjust all interior doors.

- Demo all door levers; supply and install all new interior door knobs. Finish shall be Satin Nickel.

- Demo and replace; or supply and install missing door hinges, hardware, stops or strike plates. Finish should be satin nickel.

- Demo and replace bath vanity and top. Supply and install new knobs per Spec Book. Match existing stain finish.

- Demo and replace bathroom sink faucet.

- Thoroughly scrub and clean tub and shower and re-caulk.

- Demo and replace shower trim sets.

- Demo and replace bath fan with Spec model.

- Demo and replace damaged; or supply and install missing bath accessories per Spec Book.

- Demo and replace damaged; or supply and install missing wire shelving per Spec Book.

- Service existing furnace and HRV.

- Supply and install water softener per Spec Book. Conduct water conditioning test and set hardness accordingly.

- Contractor shall complete plumbing for sump pump. Test and ensure proper function.

- Supply and install overflow pipe for hot water heater.

- Reinstall water lines for the washer. Demo and replace washer outlet box.

- Demo and replace laundry faucet.

- Provide and install window blinds throughout with Spec product. Dispose of any existing.

- Thoroughly clean all interior surfaces.

- Contractor shall have all ductwork cleaned.

- Contractor shall include 0.5% TERO tax fee in base bid. Submission of receipt for paid TERO fee is required with application for final payment.

- Construction shall be permitted through the MLBO.

Bid Alternate #1 (42938 Timber Trails Rd): Provide added cost to demo and replace all bottom window extension jambs instead of sanding and refinishing. Include cost of spray foaming after installation.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Note: Depending on bid results, the Owner reserves the right to divide the rehabs into individual projects, if it is in the Band’s financial best interest to do so.

Contacts:

Interested bidders shall contact either Carla Dunkley at 320-630-2495 or by email at carla.dunkley@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - Completed schedule of values for each home (page 2 of billing worksheet)

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Lori Trail at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: 44885 Biboon Duplex & 42938 Timber Trails

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@grcasinos.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2021 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  44885 Biboon Duplex & 42938 Timber Trails

LUMP SUM PRICE (44885 Biboon Ct):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

LUMP SUM PRICE (42938 Timber Trails Rd):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

TOTAL LUMP SUM PRICE (44885 Biboon Ct & 42938 Timber Trails Rd):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

BID ALTERNATE #1 (42938 Timber Trails bottom window extension jambs):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)                       °    LETTER FROM BONDING SURETY

- MLBO VENDOR LICENSE                                        °    SUB-CONTRACTOR LISTS (Include values)

- COPY OF CURRENT INSURANCES                        °    COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SCHEDULES OF VALUE (1 for duplex, 1 for Timber Trails)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________