Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:  11 February 2021

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:         11 March 2021

PROJECT:      39237 Little Sand Road, Hinckley, MN

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for renovation work at 39237 Little Sand Road, Hinckley, MN.   Bids will be due Wednesday, March 10, 2021 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday, March 11, 2021 at 8:30 AM.

A mandatory Pre-Bid site visit will be held at the site on Monday, February 22 at 11:00am.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description:  (Refer to 2016 MLBO Project SPEC BOOK)

Exterior;

- Exterior Log Walls: Chink exterior with Sashco “Log Builder Sealant” or acceptable alternate product to be approved by Owner.  Include specifications of the intended product as part of the alternate approval process.

- Treat wood surfaces with Borate treatment.  Primarily to provide protection from insects. Contractor to submit product data for product being used.  Contractor to follow manufacturers installation guidelines and instructions.

- Provide and install pre-finished metal Storm doors at (3) entry areas. Entry from deck, front entry and the basement entry on the back side of home.

- Utilize color match urethane caulking for joints and seams.  Provide sample colors for owner to select from to ensure proper colors.

- Demo and replace two lower level windows on rear walk-out wall. (poured concrete).  Insulate, install extension jambs and casing.  Re-trim exterior casing.  Also include exterior casing of the two upper level windows. Casing to match other windows.  Confirm locations at mandatory walk through.

- Relocate dryer vent and repair / replace rotting log at step ledger.  Remove and reinstall and or replace the necessary parts at front step and step landing as needed.

- Install seamless gutters, downspouts, leaf guards and accessories.  Provide and install gutter extensions to further direct water away from home.

- Provide and install rain cap on chimney flu pipe.

- Provide and install Drain tile and related piping around the home in an attempt to provide run-off to the west side or back of the home toward the lower elevations. This will be a daylight drain to the lower level marsh area behind the home. Currently there is assumed to be a drain system in place that was installed around the home foundation.  The existing underground drain tile and or piping is assumed to be plugged with soil and or debris as the water is not effectively shedding.  Means and methods of installing drain tile under decking and or entry areas will be determined by the Contractor and included as part of the base bid amount.  We expect the new piping / drain tile to be approximately (2) two feet underground and covered with appropriate gravel or rock to allow water into drain tile or piping.  Re-grade drainage along the exterior of the home to ensure a minimum of a (6) Six inch drop over the first (10) ten feet of distance from the home perimeter.

Interior;

- Demo existing, remove and dispose of properly and replace two electric water heaters (see SPEC BOOK) in basement level.  Include all electrical disconnects and reconnects for the water heaters. Provide and install necessary fittings, piping, connections, etc… Provide and install foam water pipe insulation for the (2) new water heaters. Water heaters installed on large diameter concrete patio block with long lasting water heater pans.

- Remove the baseboard trim at the bottom of all basement walls and provide and install new composite, pre finished baseboard trim.

- Mechanical room.  Provide and install combustion air/ passive make up air for gas fired boiler system.  Provide wall grill and 6” duct materials and installation labor complete. Remove old and provide install new sump pump and necessary piping to ensure good working order.  Repair or replace the electrical circuit and or switching complete if needed to power replacement sump pump.

- Provide and install smoke detectors and CO detectors to meet code throughout the entire home.  Ensure that all local and state requirements are met on both levels of the home.

- Provide and Install shower faucet, trim and shower head.  Restore surrounding finishes.

- Demo and replace existing stair railing and upper level guard rail system to meet code.  Replacement components to be traditional oak pre-finished components.  Homeowner to select color.

## Demo and replace all upper level flooring, except bathroom which is already complete.  1. Provide and install vinyl tread and riser covers on main stairwell from upper level to basement level.  2. Provide and install Home Depot Home Decorators Collection    5/8” Solid Tongue Groove Bamboo Flooring on the entire upper floor.    Please, include all necessary floor prep materials and installation.  Provide and install the transition strips and or expansion joints at rooms where necessary.  Confirm elevations and ensure a smooth continuous flooring installation.  Confirm expansion gap around the entire length of the perimeter.  Provide materials and install the new baseboard to cover the entire perimeter of the floors.  Stagger the end joints by no less than six inches to avoid clustering end joints.  Coordinating trim and transition pieces should match the flooring.  The Bamboo flooring will be used in the following rooms and hallways:  Upper level bedroom, hallway from front entry to kitchen area, kitchen area, deck entry on south side of home, living room, entry area near south entry. Provide samples and or literature to owner for correct choice.  Bamboo Flooring was available at Home Depot:  Internet #205124713 Model #HL615S Store SKU #1000049785Store SO SKU #1000643556.

17.Install a safety gate at the top of the stairs to provide safe passage down the hall of the main level that is at the intersection of the kitchen, living room and bed/bath hallway.

18.Provide materials and labor to repair the interior of the exterior walls in lower level.  There are multiple areas that need sheetrock patching and repair in the lower level. Match existing and adjacent wall types.

19.Remove existing Kitchen Range.  Provide and install new Kitchen Range from Home Depot; Internet #312751478 Model #NE59T4311SS Store SKU#1005258878 Store SO SKU #1005213195.

20. PROVIDE A 20 YARD CONSTRUCTION DUMPSTER AND A SEPARATE STORAGE POD FOR THIS PROJECT to accommodate owner furniture and owner property relocated for construction.  Schedule the delivery of both the 20 yard dumpster and the storage pod (2) weeks prior to commencement and schedule removal of the dumpster and storage pod (1) week after the project is completed.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact Jim Carpenter – CMD Project Coordinator at james.carpenter@millelacsband.com or at 320.364.9865 to be included

on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - A copy of Subcontractor/Material Supplier list

  - A copy of valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Lori Trail (TERO DIRECTOR) at (320) 630-2617.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: (39237 Little Sand Road  Hinckley, MN  55037)

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

- Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@grcasinos.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permit.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2021 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  39237 Little Sand Road, Hinckley, MN

LUMP SUM PRICE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #1: ()

__________________________________________________________________$____________________

(Written Value)                                                                                                 (Dollar Amount)

ALTERNATE #2 (Description):

__________________________________________________________________$____________________

(Written Value)                                                                                                 (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SUB-CONTRACTOR LISTS (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________