Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:    March 8, 2021

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:           March 31, 2021

PROJECT:    Lake Lena Community Center Remodel

TO: 	         QUALIFIED GENERAL CONTRACTORS

________________________________________________________________________________________

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for a commercial remodel project to be completed at the Lake Lena Community Center located at 45749 Grace Lake Rd., Sandstone, MN. Bids will be due Wednesday, March 31, 2021 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday April 1, 2021 at 9:00 AM.

A mandatory pre-bid site visit will be held on: Wednesday March 17, 2021 at 10:00 AM. The project is located at 45749 Grace Lake Rd., Sandstone, MN.  Directions to the site:  From Hinckley, MN: East on Hwy 48 to Grace Lake Rd.(cty 173) 23 miles.  North on Grace Lake Rd. 4 miles to the project site.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Building Code (IBC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Work Scope:

- Kitchen Space 209:

- Demo existing porcelain tile floor and cove base tile.

- Locate damaged underground waste line between floor drain and waste main line.  Saw cut concrete floor and demo slab area and subgrade to expose the damaged waste line.

- Repair damaged waste line section, bed and backfill with clean granular material.  Compact and patch concrete floor.

- Prep floor and wall base for installation of new quarry tile with integral cove base.

- Provide and install new daltile Quary Textures quarry tile flooring with integral base and epoxy grout.  Owner to select color.

- Dining Space 208:

- Demo existing VCT Flooring, clean and prep floor for new LVT installation.

- VCT area to be replaced with LVT: Shaw Contract Solitude 6”x48” plank fully adhered.

- Demo and replace existing base trim, 1x prefinished oak to match existing material.

- Demo and replace exterior aluminum storefront assemblies.  Northern door assembly to be replaced with fixed window panel.  Southern door assembly to be replaced with the same configuration.  Include new door, threshold and door hardware (same configuration).  Adjust to ensure proper operation.

- Remove loose exterior concrete, clean expansion space and patch damaged sections.  Install backer rod and caulking to fill the existing gap between the building slab and the exterior concrete patio.

- Entry Vestibule (East 110):

- Demo and replace both pairs of interior and exterior aluminum doors, including thresholds, doors and hardware in the same configuration.  Adjust to ensure proper operation.

- Gymnasium West Single HM Door:

- Demo and replace steel door, weatherstrip, lockset (storeroom function), hinges, sweep, and threshold.  Cut and remove lower side jamb rusted sections.  Install jamb patch kit, insulate, fill, prep and paint door and frame for a neat appearance.  Caulk any open gaps between frame and substrate.  Adjust door for proper operation.

- Gymnasium West Kalwall Door Assembly:

- Demo and replace steel doors, weatherstrip, panic bars, closers, hinges, sweeps and threshold.  Adjust new doors for proper operation.  Prep and paint the doors and frame assembly.  Caulk any open gaps between frame and substrate.

- Include a $10,000.00 contingency allowance in the project base bid to be used for unforeseen conditions.  Owner shall approve any use of the contingency allowance prior to commencing any repairs.

Bid Alternate #1:  None

Specified Product Substitutions:

All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact Ryan Jendro, Project Manager, at 320.630.5011 or at ryan.jendro@millelacsband.com to obtain bid form and to be included on the bidder’s list in the event that any addendums are issued for this project.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Surety Bond Letter

  - A copy of Subcontractor/Material Supplier list

  - A copy of  valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Lori Trail at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: Lake Lena Community Center Remodel Project

P.O. Box 509

Onamia, MN 56359

**Please note that the bids must be submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. **

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320) 532-8274 or via email at EThornbloom@grcasinos.com  with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- In construction contracts that are federally funded or deemed commercial, bonding is required.  These types of contracts shall demand a performance bond not less than twenty (20%) percent of the total contract price, but not to exceed $500,000.00.  A performance bond requirement is to ensure that, if a contractor defaults, the Band may request that the surety pay the expense incurred to complete the construction contract.

- In addition, all construction contracts identified as federally funded or commercial shall be covered by a payment bond equal to one payment installment or cover subcontractors/suppliers as determined by the Contracting Officer or his agents.  The payment bond must contain language stating that if the contractor fails to make a payment to its subcontractors/suppliers, the surety will make the necessary payment.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2021 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: ____________________________________________________________________________

JOB/PROJECT:    Lake Lena Community Center Remodel Project

BASE BID – (Labor and materials):

___________________________________________________________________________$____________________

(Written Value)                                                                                                                    (Dollar Amount)

ALTERNATE #1 (If Applicable)

__________________________________________________________________________$____________________

(Written Value)                                                                                                                  (Dollar Amount)

ALTERNATE #2 (If Applicable)

___________________________________________________________________________$____________________

(Written Value)                                                                                                                  (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- LETTER FROM BONDING SURETY (REQUIRED)

- COPY OF MINNESOTA CONTRACTORS LICENSE (if required)

- SUB-CONTRACTOR/SUPPLIER LIST (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: _____________________________________________________________