Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:    April 22, 2021

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:          May 19, 2021

PROJECT:      16642 & 16723 Ojibwe Drive, Onamia Rehabs

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for two (2) residential renovations located at 16642 and 16723 Ojibwe Drive, Onamia.   Bids will be due Wednesday May 19, 2021 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday May 20, 2021 at 10:00 AM.

A mandatory Pre-Bid site visit will be held on Tuesday, May 4, 2021 at 12:30 p.m. The pre-bid site visit will start at 16642 Ojibwe Drive then continue onto 16723 Ojibwe Drive. The pre-bid sites visits for Mosey Drive and Sam Drive in Isle will follow these site visits.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description: Unless otherwise noted all work shall be completed according to the 2016 MLB Spec Book.

16642 Ojibwe Drive

Exterior:

- Demo and replace all damaged; and/ or supply and install any missing siding, channel, louvered vents and accessories. All new siding shall match existing.

- Demo and replace all damaged; and/ or supply and install any missing aluminum soffit and fascia covering. All new pieces shall match existing.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms.

- Supply and install missing; and/ or demo and replace damaged window screens.

- Pressure wash, scrub and clean all siding and exterior surfaces.

- Remove and dispose of cable wires secured around the exterior of the house. Contractor shall caulk holes in the siding after removal with color matching caulk.

- Contractor shall clean and repaint post on front stoop.

- Contractor shall remove and dispose of the existing dishes installed on the roof. Contractor shall caulk screw holes or repair any damage to the roof.

- Demo and replace damaged; and/ or supply and install missing exterior light fixture components.

- Supply and install all new LED bulbs for all exterior light fixtures.

- Demo and replace damaged; or supply and install missing gutter and downspout components, including leaf guards.

- Contractor shall clean out all gutters.

- Demo and replace front entry door and frame per the Spec Book. Include brick mould and aluminum clad wrap. Demo and replace door lock set.

- Demo and replace rear entry door and frame. Include brick mould and aluminum clad wrap. Demo and replace door lock set.

- All entry door lock sets to be keyed alike. New knobs and hardware shall have Satin Nickel finish per Spec Book.

- Demo existing front storm door. Supply and install spec storm doors to front and back entries with chain kits.

- Demo and replace damaged; or supply and install missing decking, railing or spindles.

- Supply material, hardware and labor to reconstruct deck support as follows:

a. Demo existing deck posts and replace with 5”x6” posts.

b. Header shall be constructed with three 2”x 8”

c. Install two anchor bolts for each post.

- Pressure wash the deck. Stain all deck surfaces with Sherwin Williams Super Deck Exterior Waterborne solid stain per S.W. instructions. Color selection is SW 9115 Cowboy Boots.

- Supply and install missing; or demo and replace damaged window screens.

- Demo and replace damaged foundation flashing. New flashing shall match the existing.

- Repair grade around the perimeter of the home next to the foundation flashing, including installation of black dirt and seeding.

- Demo and replace damaged; or supply and install any missing outlet box covers.

- Contractor shall clean out all intake and exhaust vents.

- Provide materials and labor to construct a 5’x5’ concrete pad 4” thick. Supply and install AC-Guard security cage model AC GU with AC-Guard Top Bar to be anchored with ½”x 3” Red Head anchor sleeves to the 5’x5’ pad.

- Contractor shall move existing A/C condenser and install onto concrete pad. Contractor may need to drain, disconnect, reconnect and/ or recharge condenser.

- Supply and install any missing or damaged components to ensure proper function of exterior hose bibs.

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all damaged interior drywall surfaces. Demo and replace any sections of drywall with heavy molding. Sand any existing drywall areas of texture or seams which do not blend-in well. All (new and existing) texturing to match as closely as possible. Prime all drywall with Zinsser mold killing primer. Repaint all drywall surfaces Dover White.

- Remove and dispose of wood paneling from upper living room wall.

- Demo and replace garage service door and frame. Demo and replace door lock set. Shall be keyed alike the front and rear entry doors.

- Demo and replace any damaged; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace any damaged; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo any existing floor vents. Supply and install all new floor vents.

- Demo and replace any damaged or non-Spec; or supply and install any missing interior light fixtures with Spec products, to include all new light bulbs per Spec Book schedule.

- Install new LED bulbs throughout the interior.

- Demo all existing flooring; and replace with Congoleum Sunstone vinyl flooring. Supply and install new underlayment on upper level. Grind and prep subfloor for neat even finished appearance.

- Demo and replace all upper and lower kitchen cabinets and hardware. Supply and install island cabinet per provided Tamarack plans. Stain finish to match existing.

- Demo and replace kitchen countertops, sink, faucet and side sprayer per Spec Book.

- Demo existing and install Spec model range hood.

- Supply and install new Whirlpool oven model# WFC150MOJW and refrigerator model# WRT311FZDW.

- Demo and replace any damaged; or supply and install any missing baseboards, base, trim, jambs or casing. If existing wood is reused, it should be free of water staining, cracking and excessive paint. Match to existing finishes.

- Demo and replace any damaged; and/ or supply and install any missing window extension jambs, trim and/ or casing. Match to existing finishes.

- Demo and replace capboard in the lower front bedroom. Match existing finish.

- Demo and replace any damaged; or supply and install any missing window components.

- Thoroughly clean all window surfaces.

- Demo and replace both bath vanity tops and faucets.

- Demo and replace any damaged; or supply and install any missing vanity doors, drawers, knobs or hardware on either bath vanity. Match existing finishes.

- Demo existing and install new Spec model bath fans.

- Demo and replace both toilet stools.

- Demo existing tub/ shower surround in upper bathroom. Supply and install Warm Rain 3 piece Model: H330/331 FF.

- Demo existing shower in lower bathroom. Supply and install Warm Rain 3 piece Model: WR 533 FF.

- Demo and replace both shower valve assemblies.

- Demo and replace both shower trim sets, including showerheads.

- Demo and replace damaged; or supply and install missing bath accessories.

- Demo and replace all damaged interior pre-hung doors. Match existing finishes.

- Adjust all doors.

- Supply and install any missing door-stops throughout with spec product.

- Demo all existing interior door hardware, including: knobs, strike plates, hinges; supply and install new per Spec Book.

- Demo and replace damaged; or supply and install missing shelving. New shelving shall be wire per Spec Book.

- Remove casing from door opening between lower bedrooms. Install framing and sheetrock to enclose, thus separating the bedrooms. Finish per interior item #4. Closet opening in lower NW bedroom shall remain as a cased opening.

- Demo existing; and replace furnace with A-coil.

- Service existing HRV unit. Supply and install condensate line.

- Insulate exposed plumbing pipes in mechanical room.

- Reinstall water meter.

- Clean and repaint mechanical room floor.

- Supply and install vinyl base in mechanical room.

- Provide and install window blinds throughout with Spec product. Dispose of any existing.

- Thoroughly clean all interior surfaces.

- Contractor shall have all ductwork cleaned.

- Housing shall maintain utilities throughout construction.

- Contractor shall include 0.5% TERO tax fee in base bid. Submission of receipt for paid TERO fee is required with application for final payment.

- Construction shall be permitted through the MLBO.

16723 Ojibwe Drive

Exterior:

- Demo and replace all damaged; or supply and install any missing siding, channel, louvered vents and accessories. All new siding shall match existing.

- Demo and replace all damaged; or supply and install any missing aluminum soffit and fascia covering. All new pieces shall match existing.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms.

- Supply and install missing; or demo and replace damaged window screens.

- Pressure wash, scrub and clean all siding and exterior surfaces. This should include removal of beehives.

- Remove and dispose of cable wires secured around the exterior of the house. Contractor shall caulk holes in the siding after removal with color matching caulk.

- Supply and install vinyl wrap for post on front stoop.

- Remove and dispose of the existing dishes installed on the roof. Contractor shall caulk screw holes or repair any damage to the roof.

- Demo existing exterior light fixtures. Supply and install all new Spec Bk exterior light fixtures.

- Supply and install all new LED bulbs for all exterior light fixtures.

- Repair edge of garage roof from tree hit.

- Demo and replace damaged; or supply and install missing gutter and downspout components, including leaf guards.

- Contractor shall clean out all gutters.

- Demo any existing, supply and install concrete splashpads per Spec Book.

- Demo and replace front entry door and frame per the Spec Book, include brick mould and aluminum clad wrap. Finish per Spec Book.

- Demo patio door. Modify the opening to fit 3’ entry door per Spec Book. Supply and install spec entry door and frame, including brick mould and aluminum clad wrap. Install new lock set per Spec Book; to be keyed alike all other entry door new lock sets. After modified, finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book.

- Supply and install new lock sets for both front and rear entry doors. All entry door lock sets to be keyed alike. Knobs shall have nickel satin finish.

- Demo existing front storm door. Supply and install spec storm doors to front and back entries with chain kits.

- Demo all decking, railing, spindles, stair stringers and stair treads with only the existing deck frame to remain. Supply and install all new components and hardware to rebuild.

- Demo and replace overhead garage door metal jamb flashing and weatherstripping. Match existing color.

- Supply and install missing; and/ or demo and replace damaged window screens.

- Demo and replace damaged foundation flashing. New flashing shall match the existing.

- Repair grade around the perimeter of the home next to the foundation flashing, including installation of black dirt and seeding.

- Demo and replace any damaged; or supply and install any missing outlet box covers.

- Contractor shall clean out intake and exhaust vents.

- Provide materials and labor to construct a 5’x5’ concrete pad 4” thick. Supply and install AC-Guard security cage model AC GU with AC-Guard Top Bar to be anchored with ½”x 3” Red Head anchor sleeves to the 5’x5’ pad.

- Contractor shall move existing A/C condenser and install onto concrete pad. Contractor may need to drain, disconnect, reconnect and/ or recharge condenser.

- Replace insulation on AC condenser; and replace line insulation. Caulk around lineset opening in siding.

- Supply and install any missing or damaged components to ensure proper function of exterior hose bibs.

- Supply and install mail box and swing away post per Spec Book.

- Supply and install 9-1-1 sign above overhead garage door.

- Project Coordinator shall work with Housing to establish new electrical service and meter installation.

- Contractor shall locate water shut off and ensure proper function.

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Install sheet rock for modified rear entry door opening. Texture, paint and finish, including installation of baseboard and shoe stained to match existing finish.

- Patch, texture, prime and paint all damaged interior drywall surfaces. Demo and replace any sections of drywall with heavy molding, such as wall behind sink. Sand any existing drywall areas of texture or seams which do not blend-in well. All (new and existing) texturing to match as closely as possible. Prime all drywall with Zinsser mold killing primer. Repaint all drywall surfaces Dover White.

- Demo and replace garage service door and frame. Demo and replace door lock set. To be keyed alike the front and rear entry doors.

- Patch and tape to repair damage to garage firewall and ceiling.

- Demo and replace damaged; or supply and install missing hand rail components for upper and lower stairway, including hand rail, spindles and hardware.

- Demo and replace any damaged; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace any damaged; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo any existing floor vents. Supply and install all new floor vents.

- Demo and replace any damaged or non-Spec model; or supply and install any missing interior light fixtures with Spec products, to include all new light bulbs per Spec Book schedule.

- Install new LED bulbs throughout the interior.

- Demo any existing flooring; and replace with Congoleum Sunstone vinyl flooring. Supply and install underlayment on upper level. Grind and prep subfloor for neat even finished appearance.

- Supply and install vinyl stair treads and risers.

- Supply and install all upper and lower kitchen and island cabinets and hardware per provided Tamarack plans. Stain finish to match existing.

- Supply and install kitchen countertops, sink and faucet per Spec Book.

- Supply and install Spec model range hood.  Include ducting to exterior and conceal.

- Supply and install new Whirlpool oven model# WFC150MOJW and refrigerator model# WRT311FZDW.

- Demo and replace any damaged; or supply and install any missing baseboards, shoe, trim, jambs or casing. If existing wood is reused, it should be free of water staining, mold, cracking and excessive paint. Match to existing finishes.

- Demo and replace all window extension jambs. Include spray foam during installation. Match existing finishes.

- Demo and replace capboard on half walls in the lower level. Match existing window, baseboard, trim finishes.

- Demo and replace any damaged; or supply and install any missing window trim and/ or casing. Match to existing finishes.

- Demo and replace any damaged; or supply and install any missing window components.

- Thoroughly clean all window surfaces.

- Demo any existing; supply and install all new window blinds.

- Demo and replace all interior pre-hung doors. Match existing finishes. Front entry closet shall receive double door pre-hung unit. Hallway closets shall each receive pre-hung door unit.

- Supply and install door-stops throughout with spec product.

- Adjust all doors.

- Demo all existing interior door hardware, including: knobs, strike plates, hinges; supply and install new per Spec Book. Finish shall be satin nickel.

- Upper and lower bathrooms: For each, contractor shall:

- Supply and install bath vanity with cultured marble top per provided Tamarack plan set.

- Supply and install faucet.

- Demo and replace toilet stool.

- Demo existing tub/ shower surround in upper. Supply and install Warm Rain 3 piece Model: H330/331 FF.

- Demo existing shower in lower. Supply and install Warm Rain 3 piece Model: WR 533 FF.

- Demo and replace shower valve assemblies.

- Demo and replace shower trim set, including showerheads.

- Supply and install bath accessories per Spec Book.

- Supply and install Spec model bath fans.

- Supply and install vinyl base.

- Supply and install plumbing for lower shower, as was cut and removed. Contractor shall install drywall to close bathroom wall adjoining the mechanical room with access panel for future use. Finish per interior item #4.

- Remove and dispose of any bi-fold closet doors and hardware. Bedroom closets shall have a cased opening without doors.

- Demo and replace damaged; or supply and install missing shelving. New shelving shall be wire per Spec Book.

- Clean and paint mechanical/ laundry room floor per Spec Book.

- Supply and install vinyl base for mechanical room.

- Install new electrical service feeds, including junction boxes, if needed. Update electrical panel labeling. Supply and install panel cover. Contractor shall include any code required upgrades to restore the electrical service.

- Supply and install missing; or demo and replace damaged wiring and electrical components throughout the house.

- Demo and replace water heater per Spec Book.

- Supply and install missing; or demo and replace damaged plumbing lines throughout the home.

- Contractor shall test water system after repairing and check for leaks. Contractor shall notify Owner if any problems are discovered.

- Ensure water meter functions properly.

- Insulate exposed plumbing pipes in mechanical room per Spec Book.

- Repair or replace ductwork to reconnect existing HRV unit.

- Service existing furnace and HRV unit.

- Supply and install new furnace thermostat model: Honeywell T6 Pro Series TH6210U2001.

- Supply and install any missing batt insulation in exterior wall of the mechanical room. Demo and replace damaged; or supply and install missing poly. Supply and install sheet rock to cover exterior wall. Half wall shall have sheet rock top with no cap board installed. Finish per interior item #4.

- Thoroughly clean all interior surfaces.

- Contractor shall have all ductwork cleaned.

- Housing shall maintain utilities throughout construction.

- Include $10,000 allowance in base bid for concealed damages.

- Contractor shall include 0.5% TERO tax fee in base bid. Submission of receipt for paid TERO fee is required with application for final payment.

- Construction shall be permitted through the MLBO.

Bid Alternate #1 (16642):

Provide cost difference to:

- demo the rear entry door, modify opening and install a 36”x42” window. After modified, finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book;

- and remove and dispose of the deck;

instead of installing a 3’0 entry door with storm door and repairing the deck as listed in items # 14, 15, 16,17, 18 & 19 of the exterior work scope.

Bid Alternate #2 (16642): Provide deduct amount if Housing completes demo of the highlighted interior work scope items.

Bid Alternate #3 (16723):

Provide cost difference to:

- demo the patio door, modify opening and install a 60”x42” window. After modified, finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book;

- and remove and dispose of the deck;

instead of installing a 3’0 entry door with storm door and repairing the deck as listed in items # 16, 17, 18 & 19 of the exterior work scope.

Bid Alternate #4 (16723): Provide deduct amount if Housing completes demo of the highlighted interior work scope items.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact either Carla Dunkley at 320-630-2495 or by email at carla.dunkley@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Note: Depending on bid results, the Owner reserves the right to divide the rehabs into individual projects, if it is in the Band’s financial best interest to do so.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - Completed schedule of values for each house (billing worksheet page 2)

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Bonding Surety Letter

  - A copy of Subcontractor/Material Supplier list

  - A copy of valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Lori Trail at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: 16642 & 16723 Ojibwe Drive, Onamia

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@grcasinos.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2021 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  16642 & 16723 Ojibwe Drive, Onamia

LUMP SUM PRICE (16642 Ojibwe Drive):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

LUMP SUM PRICE (16673 Ojibwe Drive):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #1 16642 Ojibwe: (Cost to delete deck & replace patio door w/ window)

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #2 16642 Ojibwe: (Deduct if Housing Completes Highlighted Interior demo)

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #3 16723 Ojibwe: (Cost to delete deck & replace patio door w/ window)

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #4 16723 Ojibwe: (Deduct if Housing Completes Highlighted Interior demo)

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)                       °    LETTER FROM BONDING SURETY

- MLBO VENDOR LICENSE                                        °    SUB-CONTRACTOR LISTS (Include values)

- COPY OF CURRENT INSURANCES                        °    COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SCHEDULE OF VALUES (1 for each home)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________