Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:    April 22, 2021

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:          May 19, 2021

PROJECT:      43322 & 43222 Mosey Drive, Onamia Rehab

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for two residential renovations located at 43322 Mosey Drive and 43222 Mosey Drive, Onamia.   Bids will be due Wednesday May 19, 2021 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday May 20, 2021 at 10:00 AM.

A mandatory Pre-Bid site visit will be held on Tuesday, May 4, 2021 immediately following the Ojibwe Drive rehab site visits (or starting at approximately 1:15 pm). We will look at 43322 Mosey Drive first then continue onto 43222 Mosey Drive. The pre-bid site visit for Sam Drive in Isle will follow.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description: Unless otherwise noted all work shall be completed according to the 2016 MLB Spec Book.

43322 Mosey Drive

Exterior:

- Demo and replace all damaged; or supply and install any missing siding, channel, louvered vents and accessories. As existing red siding color is no longer available, contractor shall use siding from the gable end of the garage to replace damaged pieces on the rest of the home. New siding color for the garage gable end shall be colonial white. Demo existing corners and replace with colonial white one piece corners to be installed per Spec Book.

- Demo and replace all damaged; or supply and install any missing aluminum soffit and fascia covering. All new pieces shall match existing.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms.

- Supply and install missing; or demo and replace damaged window screens.

- Pressure wash, scrub and clean all siding and exterior surfaces.

- Remove and dispose of the dish and cable wires secured around the exterior of the house. Contractor shall caulk holes in the siding after removal with color matching caulk.

- Demo existing exterior light fixtures. Supply and install all new Spec Bk exterior light fixtures.

- Supply and install all new LED bulbs for all exterior light fixtures.

- Demo and replace damaged or non-matching; or supply and install missing gutter and downspout components, including leaf guards.

- Clean out all gutters.

- Demo any existing damaged concrete splashpads per Spec Book.

- Demo and replace front entry door and frame, including aluminum clad wrap and brick mould. Finish per Spec Book.

- Demo and replace rear garage service door and frame, including brick mould. Finish per Spec Book.

- Supply and install new locksets for both front entry and rear garage service door. All entry doors shall be keyed alike. Door knobs and hardware shall have Satin Nickel finish.

- Demo existing front storm door. Supply and install spec storm door with chain kit.

- Demo patio door. Modify the opening, supply and install Thermotech 60”x42” slider window. After modified, finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book.

- Demo and dispose of deck and concrete landing pad.

- Demo and replace overhead garage door metal jamb flashing and weatherstripping. Match existing color.

- Supply and install missing; and/ or demo and replace damaged window screens.

- Demo and replace damaged foundation flashing. New flashing shall match the existing.

- Supply and install any missing outlet box covers.

- Clean out intake and exhaust vents.

- Project Coordinator shall work with Housing Authority to have a new propane supply line installed.

- Provide materials and labor to construct a 5’x5’ concrete pad 4” thick. Supply and install AC-Guard security cage model AC GU with AC-Guard Top Bar to be anchored with ½”x 3” Red Head anchor sleeves to the 5’x5’ pad.

- Provide and install A/C condenser, controller and lineset.

- Supply and install any missing or damaged components to ensure proper function of exterior hose bibs.

- Supply and install mail box and swing away post per Spec Book.

- Supply and install 9-1-1 sign above overhead garage door.

- Project Coordinator shall work with Housing Authority to establish new electrical service and meter installation.

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Install sheet rock for modified rear entry door opening. Texture, paint and finish, including installation of baseboard and shoe stained to match existing finish.

- Patch, texture, prime and paint all damaged interior drywall surfaces. Demo and replace any sections of drywall with heavy molding. Sand any existing drywall areas of texture or seams which do not blend-in well. All (new and existing) texturing to match as closely as possible. Prime all drywall with Zinsser mold killing primer. Repaint all drywall surfaces Dover White.

- Demo frame for entry door from garage. Supply and install new entry door and frame with a new door lock set. To be keyed alike the front and rear garage entry doors.

- Demo and replace damaged; or supply and install missing hand rail components for upper and lower stairway, including hand rail, spindles and hardware.

- Demo and replace any damaged; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace any damaged; or supply and install any damaged or missing grilles, diffusers and return air covers with spec products.

- Demo any existing floor vents. Supply and install all new floor vents.

- Supply and install new attic access cover.

- Demo ceiling fan in living room and replace with surface mounted light fixture and controller.

- Demo any existing interior light fixtures. Supply and install all new interior light fixtures with light bulbs per Spec Book schedule.

- Install new LED bulbs throughout the interior.

- Demo all existing flooring; and replace with Congoleum Sunstone vinyl flooring. Supply and install underlayment, if needed. Grind and prep subfloor for neat even finished appearance. Existing vinyl plank is floating so existing underlayment should not have to be replaced.

- Supply and install baseshoe throughout. Match existing baseboard finish.

- Demo and replace vinyl stair treads and risers, installed properly.

- Demo and replace all lower kitchen and island cabinets. Demo and replace damaged; or supply and install missing upper cabinet components. Demo and replace toe kick for pantry cabinet. Supply and install knobs for all. Stain and hardware to match existing.

- Demo and replace kitchen and island countertops, sink and faucet with side sprayer per Spec Book.

- Demo existing range hood and replace with Spec model.  Include ducting to exterior and conceal.

- Supply and install new Whirlpool oven model# WFC150MOJW and refrigerator model# WRT311FZDW. If existing appliances are still in the house at the start of construction, the Project Coordinator will coordinate their removal by Housing Maintenance.

- Demo and replace any damaged; or supply and install any missing baseboards, trim, jambs or casing. If existing wood is reused, it should be free of water staining, mold, cracking and excessive paint. Match to existing finishes.

- Demo and replace all water stained or damaged window extension jambs. Include spray foam during installation. Match existing finishes.

- Demo and replace capboard on half walls in the lower level. Match existing window, baseboard, trim finishes.

- Demo and replace any damaged; or supply and install any missing window trim and/ or casing. Match to existing finishes.

- Demo and replace any damaged; or supply and install any missing window components.

- Thoroughly clean all window surfaces.

- Demo and replace all damaged interior pre-hung doors. Match existing finishes.

- Supply and install door-stops throughout with spec product.

- Demo all existing interior door hardware, including: knobs, strike plates, hinges; supply and install new per Spec Book. Finish shall be Satin Nickel.

- Upper and lower bathrooms: For each, contractor shall:

- Demo and replace bath vanity and top. Supply and install knobs to match those installed in kitchen.

- Demo and replace faucet.

- Demo and replace toilet stool.

- Demo and replace shower trim set, including showerhead.

- Supply and install missing bath accessories per Spec Book.

- Demo and replace bath fan. New shall be Panasonic model that will fit.

- Supply and install vinyl base.

- Remove and dispose of any bi-fold or sliding track closet doors and hardware. Repair drywall after hardware removal. Finish per interior item #4.

- Demo and replace damaged; or supply and install missing shelving. New shelving shall be wire per Spec Book.

- Clean and paint mechanical/ laundry room floor per Spec Book.

- Supply and install vinyl base for mechanical room.

- Install new electrical service feeds, including junction boxes, if needed. Update electrical panel labeling. Supply and install panel cover. Contractor shall include any code required upgrades to restore the electrical service.

- Contractor shall supply and install missing; or demo and replace damaged wiring and electrical components throughout the house.

- Demo and replace water heater per Spec Book.

- Supply and install missing; or demo and replace damaged plumbing lines throughout the home.

- Contractor shall test water system after repairing and check for leaks. Contractor shall notify Owner if any problems are discovered.

- Insulate exposed plumbing pipes in mechanical room per Spec Book.

- Service existing HRV unit.

- Reinstall laundry tub. Demo and replace faucet.

- Demo and replace furnace.

- After all repairs are complete and drywall is replaced, mechanical/ laundry room walls and ceiling shall be finished per interior item #4.

- Supply and install new furnace thermostat model: Honeywell T6 Pro Series TH6210U2001.

- Supply and install new HRV controller.

- Ensure proper function of sump pump.

- Supply and install water meter per Spec Book.

- Provide and install window blinds throughout with Spec product. Dispose of any existing.

- Thoroughly clean all interior surfaces.

- Contractor shall dispose of the 2 washers and dryer in the home.

- Contractor shall have all ductwork cleaned.

- Include $10,000 allowance in base bid for concealed damages.

- Contractor shall include 0.5% TERO tax fee in base bid. Submission of receipt for paid TERO fee is required with application for final payment.

- Housing shall maintain utilities throughout construction.

- Construction shall be permitted through the MLBO.

43222 Mosey Drive

Exterior:

- Demo and replace all damaged; or supply and install any missing siding, channel, louvered vents and accessories. All new siding shall match existing.

- Demo and replace all damaged; or supply and install any missing aluminum soffit and fascia covering. All new pieces shall match existing.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms.

- Supply and install missing; or demo and replace damaged window screens.

- Pressure wash, scrub and clean all siding and exterior surfaces.

- Remove and dispose of cable wires secured around the exterior of the house. Contractor shall caulk holes in the siding after removal with color matching caulk.

- Remove and dispose of the existing dishes installed on the roof. Contractor shall caulk screw holes or repair any damage to the roof.

- Demo existing exterior light fixtures. Supply and install all new Spec Bk exterior light fixtures.

- Supply and install all new LED bulbs for all exterior light fixtures.

- Demo and replace damaged; or supply and install missing gutter and downspout components, including leaf guards.

- Contractor shall clean out all gutters.

- Demo and replace any damaged concrete splashpads per Spec Book.

- Supply and install sheathing to repair hole cut in garage gable end wall. Finish exterior wall by installing house wrap over new sheathing. Demo white strips of siding and replace with siding to match existing.

- Demo and replace overhead garage door, metal jamb flashing and weatherstripping. Match existing colors.

- Demo and replace front, rear and exterior garage entry doors and frames per the Spec Book. Include brick mould for all; and aluminum clad wrap for front and rear entry.

- Supply and install new lock sets for both front, rear and garage entry doors. All entry door lock sets to be keyed alike. Knobs and hardware shall have Satin Nickel finish.

- Demo existing front storm door. Supply and install spec storm doors to front and back entries with chain kits.

- Demo deck and concrete landing. Contractor supply and install fill and prep area to pour 12’x12’ patio slab per Spec Book. Contractor shall install #4 re-bar reinforcement @ 24” o.c. Install expansion joint between foundation and patio slab. Contractor shall construct 4’x4’ wood landing with one (1) 4’x12” step down, to be properly anchored to the concrete patio. Rear entry door shall be centered with patio slab. Contractor shall finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book.

- Supply and install any missing outlet box covers.

- Clean out intake and exhaust vents.

- Supply and install any missing or damaged components to ensure proper function of exterior hose bibs.

- Supply and install spec AC unit condenser, controller and lineset.

- Provide materials and labor to construct a 5’x5’ concrete pad 4” thick. Supply and install AC-Guard security cage model AC GU with AC-Guard Top Bar to be anchored with ½”x 3” Red Head anchor sleeves to the 5’x5’ pad.

- Remove and dispose of existing shed and camper.

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Remove and dispose of any remaining belongings or trash within the house.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Patch, texture, prime and paint all damaged interior drywall surfaces. Demo and replace any sections of drywall with heavy molding. Texturing to match existing as closely as possible, if not already textured. Prime all drywall with Zinsser mold killing primer. Repaint all drywall surfaces Dover White.

- Demo and replace garage service door and frame. Demo and replace door lock set. To be keyed alike all entry doors.

- Remove old insulation in garage rear and gable end walls. Remove any added bracing or remains of now removed shelving in garage. Remove and dispose of window blinds.

- Demo and replace any damaged; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo and replace all grilles, diffusers and return air covers with spec products.

- Demo any existing floor vents. Supply and install all new floor vents.

- Demo and replace any damaged; or supply and install any missing interior light fixtures with Spec products, to include all new light bulbs per Spec Book schedule.

- Install new LED bulbs throughout the interior.

- Demo any existing flooring; and supply and install Congoleum Sunstone vinyl flooring and new underlayment throughout.

- Demo and replace lower kitchen and island cabinets and countertops. Demo and replace pantry cabinet. Demo and replace upper kitchen cabinet doors. Supply and install new knobs to match existing. Demo and replace damaged; or supply and install missing upper cabinet components.  Stain finish and hardware to match existing.

- Demo and replace sink, faucet and side sprayer per Spec Book.

- Demo existing range hood and replace with Spec model.

- Supply and install new Whirlpool oven model# WFC150MOJW and refrigerator model# WRT311FZDW. If old appliances are still in the house at the start of construction, Project Coordinator will contact Housing Maintenance for removal.

- Demo and replace all damaged; or supply and install missing baseboards, baseshoe, trim or casing. Match to existing finishes. Any existing with excess paint, water damage or mold that will not clean off shall be replaced.

- Demo and replace all window extension jambs. Include spray foam during installation. Match existing finishes.

- Demo and replace any damaged; or supply and install any missing window components.

- Thoroughly clean all window surfaces.

- Demo and replace all interior pre-hung doors and frames, except on bedroom closets. Demo bedroom closet doors and frames. Supply and install casing to finish openings. Match existing finishes.

- Supply and install missing door-stops.

- Supply and install all new interior door knobs with Satin Nickel finish.

- Demo and replace both bath vanities and tops. Supply and install new knobs. Match existing finishes and hardware.

- Demo and replace both bath faucets.

- Demo and replace both toilet stools.

- Thoroughly clean tub and shower and recaulk.

- Demo and replace both shower trim sets, including showerheads.

- Demo and replace bath fans with Spec model.

- Demo existing bath accessories; supply and install new per Spec Book.

- Demo any existing; and supply and install new wire shelving per Spec Book.

- Service existing furnace and install A-coil.

- Demo existing HRV unit and replace with Spec model.

- Supply and install water meter per Spec Book.

- Demo and replace sump pump, in the crawl space.

- If existing washer and dryer are still in the home at the start of construction, Project Coordinator will have removed by Housing Maintenance.

- Demo and replace laundry tub and faucet.

- Provide and install window blinds throughout with Spec product. Dispose of any existing.

- Thoroughly clean all interior surfaces.

- Contractor shall have all ductwork cleaned.

- Include $10,000 allowance in base bid for concealed damages.

- Contractor shall include 0.5% TERO tax fee in base bid. Submission of receipt for paid TERO fee is required with application for final payment.

- Housing shall maintain utilities throughout construction.

- Construction shall be permitted through the MLBO.

Bid Alternate #1 (43322): Provide deduct amount if Housing completes demo of the highlighted interior work scope items.

Bid Alternate #2 (43222): Provide deduct amount if Housing completes demo of the highlighted interior work scope items.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Note: Depending on bid results, the Owner reserves the right to divide the rehabs into individual projects, if it is in the Band’s financial best interest to do so.

Contacts:

Interested bidders shall contact either Carla Dunkley at 320-630-2495 or by email at carla.dunkley@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - Schedule of Values for Each Rehab

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - Surety Bonding Letter

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Lori Trail at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: 43322 & 43222 Mosey Drive, Onamia

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@grcasinos.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2021 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  43322 & 43222 Mosey Drive, Onamia

LUMP SUM PRICE FOR 43322 MOSEY DRIVE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

LUMP SUM PRICE FOR 43222 MOSEY DRIVE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

LUMP SUM PRICE FOR BOTH MOSEY DRIVE REHABS:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #1 43322 Mosey Drive: (Deduct if Housing Completes Highlighted Interior demo)

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #2 43222 Mosey Drive: (Deduct if Housing Completes Highlighted Interior demo)

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)                       °    LETTER FROM BONDING SURETY

- MLBO VENDOR LICENSE                                        °    SUB-CONTRACTOR LISTS (Include values)

- COPY OF CURRENT INSURANCES                        °    COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SCHEDULE OF VALUES (1 per rehab)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________