Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED:    April 22, 2021

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:          May 19, 2021

PROJECT:      3325 Sam Drive, Isle Rehab

TO: 	    QUALIFIED GENERAL CONTRACTORS

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for a residential renovation located at 3325 Sam Drive, Isle, MN 56342.   Bids will be due Wednesday, May 19, 2021 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday, May 20, 2021 at 10:00 AM.

A mandatory Pre-Bid site visit will be held on Tuesday, May 4, 2021 immediately following the Mosey Drive rehab site visits (or starting at approximately 2:30 pm).

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

Work Scope Description: Unless otherwise noted all work shall be completed according to the 2016 MLB Spec Book.

3325 Sam Drive

Exterior:

- Demo the existing tan colored siding on the rear of the house and garage; and rear half of the 2nd story. Demo any existing house wrap and install new Tyvek house wrap. If any rotten sheathing found when wrap is removed, contractor shall demo and replace as needed. For bidding purposes, contractor shall include 6 sheets in base bid. Supply and install Certainteed siding per Spec Book. Match color as close to possible to the gray siding on the front and sides of the home. Demo all existing rear siding corners and replace with one-piece corners per Spec Book in the same color as new gray siding.

- Demo and replace all damaged; and/ or supply and install any missing siding, channel, louvered vents and accessories. All new siding shall match existing gray siding.

- Demo and replace all damaged; and/ or supply and install any missing aluminum soffit and fascia covering. All new pieces shall match existing.

- Demo and replace all damaged windows components, including but not limited to panes, sashes, locking mechanisms. Clean and adjust all for proper operation.

- Supply and install missing; and/ or demo and replace damaged window screens.

- Remove and dispose of the flower box below the kitchen window.

- Pressure wash, scrub and clean all siding and exterior surfaces. Carefully remove fungus growing on shingles. Contractor shall take great care to not damage the shingles. Contractor will need be responsible for replacing any shingles that they damage.

- Remove and dispose of the existing dishes installed on the roof. Caulk screw holes or repair any damage to the roof.

- Demo all existing exterior light fixtures. Supply and install new exterior light fixtures per Spec Book.

- Supply and install all new LED bulbs for all exterior light fixtures.

- Demo all brown gutters and downspouts and replace with white gutters and downspouts. Demo and replace damaged; or supply and install missing white gutters, downspouts or leaf guards.

- Clean out all gutters.

- Demo any existing splash pads. Supply and install concrete splash pads per Spec Book.

- Demo and replace door bell.

- Demo and replace front entry door and frame per the Spec Book, including aluminum clad wrap and brick mould. Finish per Spec Book. Supply and install new lock set.

- Demo patio door. Modify the opening to fit 3’ entry door per Spec Book. Supply and install spec entry door and frame, including aluminum clad wrap and brick mould. After modified, finish this section of exterior wall by in-filling siding (color match to existing gray siding), insulation, vapor & air barriers per Spec Book. Finish per Spec Book. Supply and install new lock set.

- All entry door lock sets to be keyed alike.

- Supply and install spec storm doors to front and back entries with chain kits.

- Demo rear garage service door. After removed, finish this section of exterior wall by in-filling siding (color match to existing gray siding), insulation, vapor & air barriers per Spec Book.

- Demo toe-kick board beneath the front entry door sill. Supply and install pvc trim board to replace, shall be white in color.

- Demo and replace metal side jamb flashing and weatherstripping on both overhead garage doors. Match existing color.

- Demo and dispose of existing deck. Supply and install clean fill to level area and prep to pour 4” thick 12’x12’ concrete patio with #4 rebar reinforcement throughout on 24” o.c. grid. Install expansion joint between house foundation and patio slab. Contractor shall construct 4’x4’ treated wood landing with one (1) 4’x12” step down, to be properly anchored to the concrete patio. Rear entry door shall be centered with patio slab. Contractor shall finish this section of exterior wall by in-filling siding (color match to existing), insulation, vapor & air barriers per Spec Book.   Supply and install black dirt and seed around perimeter to help stabilize grade.

- Supply and install fill, black dirt and seeding to fill the depressed area located NW of the existing deck.

- Supply and install any missing outlet boxes and/ or covers.

- Clean out all intake and exhaust vents.

- Provide materials and labor to construct a 5’x5’ concrete pad 4” thick. Supply and install AC-Guard security cage model AC GU with AC-Guard Top Bar to be anchored with ½”x 3” Red Head anchor sleeves to the 5’x5’ pad.

- Demo and replace a/c condenser, controller and lineset per Spec Book and install onto concrete pad.

- Supply and install any missing or damaged components to ensure proper function of exterior hose bibs.

- Well – Demo and replace well pump. Contractor shall be responsible for disinfecting the existing well; and testing for coliform bacteria and nitrates.

- Demo and dispose of miscellaneous yard debris.

- Remove and dispose of the dish and cable wires secured around the exterior of the house. Caulk holes in the siding after removal with color matching caulk.

- Contractor shall have line running from telephone pole and hanging across driveway removed and disposed of.

Interior:

- Contractor shall provide dehumidification throughout renovation to minimize swelling.

- Remove and dispose of any remaining belongings or trash within the house.

- Demo and replace all smoke detectors with spec combo smoke c/o detector to meet code compliance.

- Install sheet rock for modified rear entry door opening. Texture, paint and finish, including installation of baseboard and shoe stained to match existing finish.

- Patch, texture, prime and paint all damaged interior drywall surfaces. Demo and replace any sections of drywall with heavy molding. Sand any existing drywall areas of texture or seams which do not blend-in well. All (new and existing) texturing to match as closely as possible. Prime all drywall with Zinsser mold killing primer. Repaint all drywall surfaces Dover White.

- Demo and replace frame and entry door to garage. Demo and dispose of existing storm door. Demo and replace door lock set. To be keyed alike all entry doors.

- Ensure handrail for steps inside garage is secure. Demo and replace any damaged components. Secure any loose railing or handrail components.

- Remove and dispose of shelving and miscellaneous debris in garage. Demo and dispose of poly hanging down in ceiling of garage.

- Remove and dispose of garage door opener.

- Demo and replace any damaged; or supply and install any missing, damaged or painted receptacle and switch covers.

- Demo all existing; supply and install grilles, diffusers, return air covers and floor vents with spec products.

- Demo and replace any damaged; or supply and install any missing interior light fixtures with Spec products. All light fixtures shall be per Spec Book schedule.

- Demo ceiling fan and controller. Supply and install surface mounted fixture per Spec Book.

- Install new LED bulbs throughout the interior.

- Demo all existing flooring. Supply and install Congoleum Oasis Beige vinyl flooring. Supply and install underlayment where needed. Grind and prep subfloor for neat even finished appearance.

- Supply and install vinyl stair treads and risers.

- Supply and install handrail for lower stairs.

- Demo and replace all kitchen cabinets and countertops. New countertop selection: Concrete Stone 7267-58. Stain and hardware finish to match existing. New cabinets shall have drawer and door pulls per Spec Book.

- Demo and replace kitchen sink, faucet and side sprayer.

- Demo and replace range hood per Spec Book. Include venting to the exterior and conceal.

- Supply and install new Whirlpool oven model# WFC150MOJW and refrigerator model# WRT311FZDW.

- Demo and replace any damaged; or supply and install any missing baseboards, baseshoe, trim, jambs or casing. Match to existing finishes. Woodwork should be free of water staining, mold and excessive paint.

- Demo bi-fold closet doors and hardware. Supply and install casing for closet openings.

- Demo and replace all interior pre-hung doors and frames (except bedroom closets). Bedroom closets shall remain as cased openings after doors are removed. Match existing finishes.

- Supply and install any missing door-stops throughout with spec product.

- Demo and replace all interior door hardware, including: knobs, strike plates and hinges per Spec Book. All new knobs and hardware shall have Satin Chrome finish.

- Demo and replace all window extension jambs per Spec Book. Match existing stain color. Include spray foam after installation.

- Demo and replace any damaged; or supply and install any missing window components.

- Thoroughly clean all window surfaces.

- Demo any existing; supply and install all new window blinds per Spec Book.

- Lower level living room:

- Remove wood paneling from walls and around windows.

- Remove wall mirror.

- Make any needed repairs to sheet rock. Texture, prime and paint per interior item #5. Half wall shall have sheetrock finished top.

- Supply and install wood window casing per Spec Book. Match finish of casing on upper floor bedroom windows.

- Remove track lighting. Supply and install surface mounted light fixture per Spec Book.

- Supply and install baseboard and shoe to match the rest of the house.

- Laundry room:

- Remove wood from walls.

- Make any needed repairs to sheet rock. Texture, prime and paint per interior item #5. Half wall shall have sheetrock finished top.

- Supply and install vinyl base. Selection: 29 Moon Rock

- Demo and replace laundry tub and faucet per Spec Book.

- Lower bathroom:

- Demo and replace toilet stool.

- Demo bath accessories and replace per Spec Book.

- Demo and replace shower trim set, including showerhead.

- Thoroughly clean and recaulk shower surround.

- Demo and dispose of shelving.

- Demo and replace bath vanity, top, and faucet per Spec Book. Top shall be cultured marble per Spec Book.

- Demo and replace bath fan.

- Mechanical room:

- Secure loose hanging cable/ tube above entry.

- Spray foam and caulk around window, as needed to eliminate draft and provide for snugger fit.

- Install new whole house filter.

- Reinstall water softener; sanitize; and add salt.

- Complete water conditioning test and set softener hardness accordingly.

- Service existing furnace and HRV unit.

- Insulate exposed plumbing pipes.

- Clean and paint mechanical room floor. Supply and install vinyl base.

- Supply and install sheetrock and tape to enclose wall and ceiling.

- Main bathroom:

- Demo and replace toilet stool.

- Demo bath accessories and replace per Spec Book.

- Demo and replace shower trim set, including showerhead.

- Demo and replace shower valve assembly.

- Demo existing tub/ shower. Supply and install H330/331 FF Tub/Shower.

- Demo and replace sink faucet per Spec Book.

- Demo and replace bath fan.

- Remove and dispose of decorative ceiling cover. Texture, prime and paint ceiling per interior item #5.

- Demo and replace damaged; or supply and install missing shelving. New shelving shall be wire per Spec Book.

- Thoroughly clean all interior surfaces.

- Contractor shall have all ductwork cleaned.

- Include a $10,000 contingency allowance to be used at Project Coordinator’s discretion for needed repairs discovered after the project has commenced.

- Contractor shall include 0.5% TERO tax fee in base bid. Submission of receipt for paid TERO fee is required with application for final payment.

- Housing will maintain utilities throughout construction.

- Construction shall be permitted through the MLBO.

Bid Alternate #1: Contractor shall provide cost difference to complete the following instead of demolishing deck and replacing with concrete slab.

- Demo and replace all damaged: decking, stair treads, top rails, handrails and spindles.

- Remove and dispose of stairs on north side of deck.

- Supply and install top rail, handrail and spindles to in-fill opening left by stair removal.

Bid Alternate #2: Contractor shall provide unit cost for replacing rotten sheathing on exterior walls. Unit cost per sheet shall include all labor and materials required to remove old and install new sheet. Unit cost shall be utilized to add or deduct the cost of actual sheets replaced.

Bid Alternate #3: Contractor shall provide lump sum cost to demo and replace furnace. Include demo of existing thermostat and replacing with: Honeywell T6 Pro Series TH6210U2001.

Bid Alternate #4: Provide deduct amount if Housing completes demo of the highlighted interior work scope items.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact either Carla Dunkley at 320-630-2495 or by email at carla.dunkley@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - Completed schedule of values (billing worksheet page 2).

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of  valid State of Minnesota Contractor’s License

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Lori Trail at (320) 532-4778.

All proposals must be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: 3325 Sam Drive Rehab

PO Box 509

Onamia, MN 56359

**Please note that the bids submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. Do not require delivery signature.**

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@grcasinos.com with questions regarding licensing and for the license application.

PERMIT AND CONTRACTOR REQUIREMENTS:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.  Please note that bonding requirements on Mille Lacs Band Funded Residential projects has changed.  All projects over $50,000 require bonding regardless of company ownership.

MLBSA Section 17 Procurement Statue Ordinance 03-06 states the following:

Section 17. Bonding

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2021 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT:  3325 Sam Drive Rehab

LUMP SUM PRICE (3325 Sam Drive):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

BID ALTERNATE #1 (Deck Repairs):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

BID ALTERNATE #2 (Ext. Sheathing Unit Cost):

_______________________________________________________/ per sheet     $______________/ per sheet

(Written Value)                                                                                               (Dollar Amount)

BID ALTERNATE #3 (Furnace Replacement):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

ALTERNATE #4 (Deduct if Housing Completes Highlighted Interior demo):

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)                       °    LETTER FROM BONDING SURETY

- MLBO VENDOR LICENSE                                        °    SUB-CONTRACTOR LISTS (Include values)

- SCHEDULE OF VALUES                                          °    COPY OF MINNESOTA CONTRACTOR’S LICENSE

- COPY OF CURRENT INSURANCES

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________