Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED: 12 October 2017

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:        1 November 2017

PROJECT:     D3 Softball Dirt-Dugout-Bleachers

TO: 	    Qualified General Contractors & Concrete Professionals

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed lump sum bids for Softball field infield Dirt, Dugout slab and bench, bleacher slab and bench installation at the D3 Softball field located within the Zhingwaak Oodena Phase 3 subdivision in Hinckley, MN. Bids will be due Wednesday 1 November October 2017 at 3:00 PM. Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday 2 November 2017 at 8:30 AM.

A mandatory pre-bid site visit will be held on: Wednesday October 18th at 1:30 PM at the softball located just South of the Grand Casino Hinckley Hotel entrance off of Pine County. Rd. #15 (Hinckley Rd), Hinckley, MN.

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Building Code (IBC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- Contractor shall be responsible for all debris removal related to all work performed under this work scope.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Work Scope:

- Remove 1” of existing skinned infied dirt.

- Provide and install 29 Tons of Diamond Brand Red Infield Conditioner.

- Diamond Red Infield Conditioner Local Supplier:

D. Ervasti Sales Co.,LLC

320-654-9693

866-945-DIRT(3478)

320-267-9439 cell

e-mail:  dervasti@charter.net

- Pour at each dugout location a 24ft long, 12ft concrete pad 4” thick to be reinforced with #4 rebar @ 48” o.c.

- Pour for bleachers an 8ft deep by 16 feet long concrete pad 4” thick to be reinforced with #4 rebar @ 48” o.c.

From http://www.anthem-sports.com

- Install at each dugout location a 21ft long aluminum bench with backrest.

  - Permanent bench with backrest

  - 2'' round galvanized steel legs

  - 10'' wide anodized aluminum planks

  - Planks are finished with end caps

  - Seat plank is 17.5" from ground

  - 5 year warranty

- Provide and install a 3 Row, 15' STANDARD Aluminum Bleacher

  - 3 row, 15' long

  - Seats 30 people

  - Total height: 2' 4"

  - Total depth: 4' 1.5"

  - 2"x12" wide anodized seat planks

  - 2"x10" single mill finish foot planks

  - Aluminum frame construction

- Provide and install (2) Schutt Hollywood Step Down Pitching Rubbers, ADULT, 24"

- Provide and install Schutt Hollywood Bury All Home Plate

- Schutt Jack Corbett MLB Hollywood Bases, Set of 3, 12901010

- Provide and install (2) Vertical Baseball/Softball Distance Markers at foul poles

- Provide and install Poly-Cap Fence Top Protector, 250' Length (2)

- Provide and install (2) sets of Schutt Base Ground Anchors, set of 3

- Provide and install Rubber Base Plug for Baseball Ground Anchors

- Provide Big League Infield Nail Drag, 5'W x 3'L

- Provide Jaypro FMP-10OFF Basic Baseball Field Maintenance Pkg, OFFICIAL (ADULT)

- Contractor shall be responsible for on-site utility identification, prior to installation.

- Contractor shall be responsible for permitting through MLBO and through the City of Hinckley.

Specified Product Substitutions:  All proposed product substitutions must be approved by the Project Coordinator 5 days prior to bid.  Any approved substitutions, will be made available to all bidders.

Contacts:

Interested bidders shall contact Mike Moilanen, Director of Planning and Project Management, at mike.moilanen@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within 21 days of the receipt of Notice to Proceed.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - A copy of Subcontractor/Material Supplier list

  - A copy of  valid State of Minnesota Contractor’s License (if applicable)

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Craig Hansen at (320) 532-4778.

All proposals MUST be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: D3 Softball Dirt-Dugout-Bleachers

P.O. Box 509

Onamia, MN 56359

**Please note that the bids must be submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. **

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

- Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Jacquelyn Smith at the Business Regulations Office at (320)532-8240 or by email at jsmith@mlcv.com with questions regarding licensing and for the license application.

Permit and Contractor Requirements:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits and City of Hinckley Fencing Permit.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2018 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: ____________________________________________________________________________

JOB/PROJECT:    D3 Softball Dirt-Dugout-Bleachers

LUMP SUM PRICE:

(Labor and materials):

___________________________________________________________________________$____________________

(Written Value)                                                                                                                    (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- SUB-CONTRACTOR/SUPPLIER LIST (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: _____________________________________________________________