Community Development

REQUEST FOR PROPOSAL

OWNER:        MILLE LACS BAND OF OJIBWE		DATE ISSUED: April 3, 2018

43408 OODENA DRIVE

ONAMIA, MN 56359                   		BID DATE:        April 18, 2018

PROJECT:     	East Lake Pow Wow Playground– McGregor, MN

TO: 	    Qualified Playground Equipment Contractors

THIS IS NOT A CHANGE ORDER NOR A DIRECTION TO PROCEED WITH THE WORK DESCRIBED HEREIN.

The Mille Lacs Band of Ojibwe, Community Development office will be accepting sealed bids for purchase and installation of playground equipment for a playground to be located at the Mille Lacs Band of Ojibwe District II Pow Wow Grounds in McGregor, MN.  Bids will be due Wednesday 18 April 2018 at 3:00 PM.  Bids received will be opened and qualified by the Mille Lacs Band of Ojibwe on Thursday 12 April 2018 at 8:30 AM.

A mandatory pre-bid site visit will be held Monday, April 9th at 1:00 pm.

Directions to project site:  From City of McGregor: Hwy 65 South 3.2 miles.  Project site is on the East side of Hwy 65 just south of Aitkin County Rd. 30.  There is a driveway leading to an existing garage structure. (37736 State Hwy 65, McGregor)

General Notes:

- It is the Contractors responsibility to identify any and all discrepancies in the scope of work, not meeting Industry Standards or that which is inconsistent with the International Residential Code (IRC), and Mille Lacs Band of Ojibwe 2016 MLB Project Specification Book.

- All electrical wiring, apparatus and equipment for electric light, heat and power, technology circuits or systems shall comply with the rules of the Department of Commerce or the Department of Labor and Industry, as applicable, and be installed in conformity with accepted standards of construction for safety to life and property.

- Contractor must reconnect all utilities, service panel or service feed. Also includes gas, propane.  Include such equipment or materials identified in the pre-bid conference.

- Contractor will secure all permits and fees.

- Contractor is responsible for a thorough investigation of the scope of work.

- Contractor will repair any damage to the property or structure created by the scope of work.

- Contractor shall be responsible for all debris removal related to all work performed under this work scope.

- NO WORK SHALL BE PERFORMED UNTIL ALL REQUIRED PERMITS HAVE BEEN ISSUED AND COPIES IN THE POSSESSION OF THE PROJECT COORDINATOR AND /OR MILLE LACS BAND BUILDING OFFICIAL.

COMMUNITY DEVELOPMENT WILL, TO THE GREATEST EXTENT FEASIBLE, GIVE PREFERENCE IN THE AWARD OF CONTRACT TO INDIAN ORGANIZATIONS AND INDIAN-OWNED ECONOMIC ENTERPRISES.

Work Scope Description:

- Playground equipment needed for age group(s) 2 to 10 years.

- Budget amount $40,000.00.

- Please submit 2-3 designs for review.  From all submitted proposals, Submit separate bid form for each of your proposals.

- Playground base site work will be performed by Owner: (includes rock base, draintile timber boarder, geotextile filter fabric installation and wood mulch installation)

- Playground Equipment Suppliers shall submit proposals for playground equipment, 9” of wood mulch (supply and deliver only), and geotextile filter fabric (supply and deliver only).

- Provide Product data for all equipment being proposed, and cost breakdown for all items.

- All Contractors and Suppliers must be licensed with the Mille Lacs Band of Ojibwe.  Please contact MLBO Business Regulations office at 320-532-8859 for more information.

- Project Location is McGregor, MN.

- Davis Bacon Construction wage rates apply.

- All proposals must be Sealed Bid Form.

- Include product order lead time in your proposal.

- Final color selections will be made by the Owner after award.  Provide color package templates.  Once selected, contractor shall produce selected color drawings for final approval.

Contacts:

Interested bidders shall contact Archie Cash at 320-532-7432 or by email at archie.cash@millelacsband.com to be included on the bidder’s list in the event that any addendums are issued for this project.

Mobilization:

- The Contractor shall be capable of mobilizing his equipment and crews within seven days of the receipt of Notice to Proceed.

- Contractor shall submit building schedule to Project Coordinator at the time of contract signature by the Contractor.

- Contractor shall provide means and methods for all building phases of construction.

COMMUNITY DEVELOPMENT/PROJECT MANAGEMENT RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS FOR ANY REASON.

Bidding notes:

- Submit proposal in lump sum (supply and install), not to exceed amount

- All Contractors (including subcontractors) must comply with Davis Bacon wage requirements.

- All Contractors must provide the following along with their bid submittal:

  - Completed and signed MLB Community Development Construction Bid Form

  - A copy of Current MLB Vendor’s License (or a copy of the submitted application)

  - A copy of Current Insurance Certificate

  - Completed and signed copy of the TERO Compliance Plan

  - A copy of Subcontractor/Material Supplier list

  - A copy of Authorized Signature Sheet (submitted with first bid submittal)

- All Contractors must comply with all Mille Lacs Band of Ojibwe American Indian Employment requirements (see 18 MLBSA § 5). Contact Craig Hansen at (320) 532-4778.

All proposals MUST be mailed and labeled as follows:

Mille Lacs Band of Ojibwe

Commissioner of Community Development

Sealed bid: East Lake Playground equipment

P.O. Box 509

Onamia, MN 56359

**Please note that the bids must be submitted via mail to the P.O. Box. FedEx and UPS will not deliver to a P.O. Box and the Onamia post office will not accept hand delivered items. Please plan accordingly to ensure the timely receipt of your bid submittal. **

**The Band reserves the right to reject any bid that it is unable to collect at the Onamia post office by the bid deadline date and time, provided that the Band has made diligent and reasonable efforts to collect the bid. The Band reserves this right even in the event that the bid has been postmarked before the deadline.

PROPOSALS NOT SUBMITTED IN THIS MANNER WILL BE REJECTED.

Licensing:

- Firms must be licensed with the Mille Lacs Band of Ojibwe.  A copy of this license (or the license application) must accompany each bid. Licensing process can take several weeks. If you are not currently licensed with the MLBO, please submit a copy of your license application along with your proposal. Contact Elizabeth Thornbloom at (320)532-8274 or via email at EThornbloom@mlcv.com with questions regarding licensing and for the license application.

Permit and Contractor Requirements:

Permits:  Contractors are responsible to attain all necessary permits for all work, including Mille Lacs Band of Ojibwe (MLBO) Permits.  Mille Lacs Band Permit Fee is $50.00.

Bonding Requirements:  In accordance with 2016 MLB Project Specification Book.

- In construction contracts that are federally funded or deemed commercial, bonding is required.  These types of contracts shall demand a performance bond not less than twenty (20%) percent of the total contract price, but not to exceed $500,000.00.  A performance bond requirement is to ensure that, if a contractor defaults, the Band may request that the surety pay the expense incurred to complete the construction contract.

- In addition, all construction contracts identified as federally funded or commercial shall be covered by a payment bond equal to one payment installment or cover subcontractors/suppliers as determined by the Contracting Officer or his agents.  The payment bond must contain language stating that if the contractor fails to make a payment to its subcontractors/suppliers, the surety will make the necessary payment.

- For all Band funded residential construction projects, a performance bond is required for contracts in excess of $50,000.00.  The performance bond shall be at a minimum twenty (20%) percent of the contract price, but not in excess of $500,000.00.

SECTION II – BIDDING FORMS

Bidding Requirements and Contract Forms

COMMUNITY DEVELOPMENT

PROJECT MANAGEMENT

FY 2018 CONSTRUCTION BID FORM

REQUIRED FOR ALL BIDS

FIRM NAME: __________________________________________________________________________

JOB/PROJECT: East Lake Pow Wow Playground equipment

LUMP SUM PRICE:

__________________________________________________________________$____________________

(Written Value)                                                                                               (Dollar Amount)

Acknowledgement of Addendum(s): 1) __________ date    2) ___________date   3) ___________date

BID GUARANTEE PERIOD:

I agree to hold this bid open for a period of 90 days after the bid opening.  If this bid is accepted I agree to execute a Contract and/or a Purchase Order with the Mille Lacs Band of Ojibwe along with furnishing all required bonding (if required) and insurances.

TERO COMPLIANCE:

I understand that this company, its subcontractors and all employees performing work on this project will be expected to comply with all Mille Lacs Band TERO Compliance Regulations. Upon being informed that I will be awarded a contract for this project, I will submit all required TERO Compliance Plans directly to the MLB TERO Office for review and approval.

Acknowledgement of TERO Compliance:  _________________________

ATTACHMENTS REQUIRED:  Failure to provide any of these attachments will result in bid disqualification.

- MLB BID FORM (MUST BE SIGNED)

- MLBO VENDOR LICENSE

- COPY OF CURRENT INSURANCES

- LETTER FROM BONDING SURETY

- COPY OF MINNESOTA CONTRACTOR’S LICENSE

- SUB-CONTRACTOR LISTS (Include values)

NAME: ________________________________________TITLE:________________________

SIGNATURE: _______________________________________DATE:____________________

FIRM NAME: ________________________________TELEPHONE:_____________________

ADDRESS: ____________________________________________________________________

EMAIL ADDRESS: ____________________________________________________________